OpenSoC Tier-1

Installation Guide

Step-by-step setup for new users on Windows 10/11 and WSL2 Ubuntu
QuestaSim + RISC-V GCC + Python · Shakti E-class RV32IM SoC

IIITDM Chennai  ·  OpenSoC Educational SoC Programme  ·  Rev 1.0  ·  April 2026

# 1.  Overview

This guide walks a first-time user through installing every tool needed to compile and simulate the OpenSoC Tier-1 Shakti E-class SoC.  Follow every step in order.  Each step ends with a verification command so you can confirm success before continuing.

The simulation stack is: RISC-V GCC (firmware) → Python eclass_sim.py (runner) → QuestaSim vlog/vopt/vsim (RTL compile + simulate).  No Verilog skills are needed to run the pre-built tests.

# 2.  System Requirements

| Item | Minimum | Recommended |
| --- | --- | --- |
| OS | Windows 10 (2004+) | Windows 11 22H2+ |
| RAM | 8 GB | 16 GB |
| Disk | 5 GB free | 20 GB free |
| QuestaSim | 2020.4+ | 2023.1 / 2024.1 |
| Python | 3.8+ | 3.11+ |
| RISC-V GCC | any riscv32/64 elf | xpack 14.2.0 |

Linux / WSL2: All steps work identically.  Substitute PowerShell $env: syntax with export bash syntax where noted.

Step 1 — Get the Repository

Open PowerShell (Windows) or bash (WSL2 / Linux):

# Windows PowerShell

cd C:\Users\YourName\Projects

git clone https://github.com/your-org/opensoc-tier1.git

cd opensoc-tier1\soc1.1

If you received a zip archive:

Expand-Archive opensoc-tier1.zip .

cd opensoc-tier1\soc1.1

Verify — dir (Windows) or ls (bash) should show:

CLAUDE.md   INSTALL.md   README_ECLASS.md   rtl/   verif/   sw/   scripts/

Step 2 — Install QuestaSim

Download Intel Quartus Prime Pro from the Intel FPGA Download Center.  During installation select QuestaSim-Intel FPGA Pro Edition (questa_fse).

Common install paths:

| Version | Default Path |
| --- | --- |
| 23.1 | C:\intelFPGA_pro\23.1\questa_fse |
| 22.4 | C:\intelFPGA_pro\22.4\questa_fse |
| 21.1 | C:\intelFPGA_pro\21.1\questa_fse |
| Standalone | C:\questasim64_2023.3 |

Verify QuestaSim after installation:

& "C:\intelFPGA_pro\23.1\questa_fse\win64\vsim.exe" -version

# Expected:

QuestaSim-64 vsim 2023.1 Compiler 2023.01 Jan 27 2023

Step 3 — Set QUESTA_HOME

Setting QUESTA_HOME is the most reliable way to point eclass_sim.py at your QuestaSim installation.

Windows — set for current session only:

$env:QUESTA_HOME = "C:\intelFPGA_pro\23.1\questa_fse"

Windows — set permanently (survives reboot):

[System.Environment]::SetEnvironmentVariable(

"QUESTA_HOME",

"C:\intelFPGA_pro\23.1\questa_fse",

"User")

Open a NEW PowerShell window, then verify:

echo $env:QUESTA_HOME

# Expected: C:\intelFPGA_pro\23.1\questa_fse

Test-Path "$env:QUESTA_HOME\win64\vsim.exe"

# Expected: True

WSL2 / Linux:

export QUESTA_HOME="/opt/questasim"

echo 'export QUESTA_HOME="/opt/questasim"' >> ~/.bashrc

Auto-detection: if QUESTA_HOME is not set, eclass_sim.py searches the following paths in order:

| Priority | Path | Notes |
| --- | --- | --- |
| 1 | $QUESTA_HOME (if set) | Use when set — highest priority |
| 2 | C:\intelFPGA_pro\23.1\questa_fse | Intel Quartus Pro 23.1 |
| 3 | C:\intelFPGA_pro\22.4\questa_fse | Intel Quartus Pro 22.4 |
| 4 | C:\modeltech64_2023.3 | Siemens standalone 2023.3 |
| 5 | C:\questasim64_2023.3 | Mentor standalone 2023.3 |
| 6 | /opt/questasim | Linux / WSL2 |
| 7 | vsim on PATH | Fallback — lowest priority |

Step 4 — Install RISC-V GCC Toolchain

Firmware source files (C) must be compiled to a Verilog hex file before simulation.  Choose one of the three options below.

Option A: xpack RISC-V GCC (recommended for Windows)

1. Download the win32-x64 zip from the xpack GitHub releases page (search "xpack-dev-tools/riscv-none-elf-gcc-xpack/releases").

2. Extract to a permanent location, e.g. C:\tools\xpack-riscv-none-elf-14.2.0

3. Set the RISCV environment variable:

# Session only

$env:RISCV = "C:\tools\xpack-riscv-none-elf-14.2.0"

# Permanent

[System.Environment]::SetEnvironmentVariable("RISCV",

"C:\tools\xpack-riscv-none-elf-14.2.0", "User")

4. Open a new PowerShell window and verify:

& "$env:RISCV\bin\riscv-none-elf-gcc.exe" --version

# Expected:

riscv-none-elf-gcc (xPack GNU RISC-V Embedded GCC x86_64) 14.2.0

Option B: WSL2 Ubuntu (apt package)

sudo apt update

sudo apt install -y gcc-riscv64-unknown-elf binutils-riscv64-unknown-elf

riscv64-unknown-elf-gcc --version

# Expected: riscv64-unknown-elf-gcc (Ubuntu ...) 12.2.0

Toolchain search order (sw/Makefile):

| Priority | Binary | Source |
| --- | --- | --- |
| 1 | riscv32-unknown-elf-gcc | xpack or crosstool-ng — exact RV32 target |
| 2 | riscv-none-elf-gcc | Newer xpack naming convention |
| 3 | riscv64-unknown-elf-gcc | Ubuntu apt — multilib; also builds RV32IM |

Step 5 — Install Python 3.8+

Windows:

Download the installer from python.org/downloads and run it.  Check "Add Python to PATH" during installation.

python --version

# Expected: Python 3.x.x  (3.8 or later)

# Install python-docx (for document generation only)

pip install python-docx

WSL2 / Ubuntu:

sudo apt install -y python3 python3-pip

pip3 install python-docx

python3 --version

python-docx is only required to regenerate the Word documents.  It is NOT needed to run simulations.

Step 6 — Verify All Tools

Run this checklist before building firmware:

# 1. Python

python --version

# 2. QuestaSim

& "$env:QUESTA_HOME\win64\vsim.exe" -version

# 3. RISC-V GCC

& "$env:RISCV\bin\riscv-none-elf-gcc.exe" --version

# 4. make

make --version

make on Windows: Use Git Bash (ships with make) or MSYS2.  Alternatively, python scripts/build_sw.py is a pure-Python fallback that does not need make.

Step 7 — Build Firmware

Compile all four test programs to Verilog hex format.  Run from the repo root in Git Bash, WSL2, or any shell with make:

make -C sw

# Or individual targets

make -C sw gpio_l1

make -C sw gpio_l2

make -C sw uart_l1

make -C sw uart_l2

Expected output:

[CC]  gpio_test.c → gpio_test_l1.elf

[HEX] gpio_test_l1.elf → sw/tests/gpio_test_l1.hex

[CC]  gpio_test_l2.c → gpio_test_l2.elf

[HEX] gpio_test_l2.elf → sw/tests/gpio_test_l2.hex

[CC]  uart_test.c → uart_test_l1.elf

[HEX] uart_test_l1.elf → sw/tests/uart_test_l1.hex

[CC]  uart_test_l2.c → uart_test_l2.elf

[HEX] uart_test_l2.elf → sw/tests/uart_test_l2.hex

Verify hex files:

ls sw/tests/

# gpio_test_l1.hex  gpio_test_l2.hex  uart_test_l1.hex  uart_test_l2.hex

If make is not available (Windows without Git Bash):

python scripts/build_sw.py --test gpio_l1

python scripts/build_sw.py --test gpio_l2

python scripts/build_sw.py --test uart_l1

python scripts/build_sw.py --test uart_l2

Step 8 — Run Your First Simulation

eclass_sim.py handles the complete flow (vlog → vopt → vsim) and auto-rebuilds firmware if any hex file is missing or stale:

python scripts/eclass_sim.py gpio_l2

Expected transcript (abbreviated):

[SIM] Building firmware: make -C sw gpio_l2

[SIM] vlog  → build/eclass_gpio_l2/compile.log

[SIM] vopt  → build/eclass_gpio_l2/vopt.log

[SIM] vsim  → build/eclass_gpio_l2/sim.log

[TB] Reset released — ISS booting from 0x00010000

[TB] PC = 0x00010000  (Boot ROM entry)

[TB] PC = 0x80000000  (SRAM entry, after boot jump)

[TB] GPIO walk-1 pattern: 32 iterations PASS

[TB] GPIO walk-0 pattern: 32 iterations PASS

[TB] GPIO_IN OR-accumulation PASS

[TB] TOHOST written after 230 cycles: 0x00000001

[TB] PASS -- all checks passed [L2]

[SIM] PASS  (gpio_l2)

A [TB] PASS line and exit code 0 confirm the simulation passed.  TOHOST = 0x1 means the CPU firmware wrote the PASS sentinel.

Step 9 — Run Full Regression

Available tests:

| Test Name | Level | What It Tests |
| --- | --- | --- |
| gpio_l1 | Level-1 | CPU drives 16-bit GPIO walk-1/0 via OpenTitan gpio.sv |
| gpio_l2 | Level-2 | CPU drives 32-bit GPIO walk-1/0 + patterns via t1_periph_ss_32 |
| uart_l1 | Level-1 | CPU sends 4-byte loopback (0x55/0xAA/0x00/0xFF) via OpenTitan uart.sv |
| uart_l2 | Level-2 | CPU sends 4-byte loopback via t1_periph_ss_32 at baud_div=10 |
| cov_l2 | Level-2 | Functional coverage run — 26 covergroups; use with --coverage |

Run all tests:

# Level-1 (quick, ~2 minutes)

python scripts/eclass_sim.py gpio_l1

python scripts/eclass_sim.py uart_l1

# Level-2 (full, ~5 minutes)

python scripts/eclass_sim.py gpio_l2

python scripts/eclass_sim.py uart_l2

python scripts/eclass_sim.py cov_l2 --coverage

Or using the Makefile (WSL2 / Git Bash):

make -f verif/scripts/Makefile.questa eclass_regression      # Level-1

make -f verif/scripts/Makefile.questa eclass_regression_l2   # Level-2

Expected final lines:

GPIO L2: PASS

UART L2: PASS

[COV] Run complete — ucdb in build/eclass_cov_l2/coverage/

Step 10 — Optional — GUI, Waves, Coverage

# Open QuestaSim GUI with waveform window

python scripts/eclass_sim.py gpio_l2 --gui --waves

# Run with coverage collection

python scripts/eclass_sim.py cov_l2 --coverage

# Open coverage report:

# build/eclass_cov_l2/coverage_html/index.html

# 3.  Build Artifacts

After a successful run, build/eclass_<test>/ contains:

| File / Path | Contents |
| --- | --- |
| compile.log | vlog output — errors and warning counts |
| vopt.log | vopt elaboration messages |
| sim.log | vsim transcript — look for [TB] PASS |
| <test>.wlf | Waveform database (--waves only) |
| coverage/ | UCDB coverage database (--coverage only) |
| coverage_html/index.html | HTML coverage report (--coverage only) |

# 4.  Command Reference

| Command | Description |
| --- | --- |
| python scripts/eclass_sim.py <test> | Full flow: auto-build firmware + compile + simulate |
| python scripts/eclass_sim.py <test> --clean | Wipe build dir first, then simulate |
| python scripts/eclass_sim.py <test> --gui --waves | Open QuestaSim GUI with waveform window |
| python scripts/eclass_sim.py <test> --no-build | Skip firmware rebuild (use existing hex) |
| python scripts/eclass_sim.py <test> --verbose | Print vlog/vsim output to console in real time |
| python scripts/build_sw.py --test <name> | Build firmware only (no make required) |
| make -C sw | Build all 4 firmware hex files |
| make -C sw gpio_l2 | Build a single hex target |
| make -f verif/scripts/Makefile.questa eclass_regression | Level-1 regression (WSL2/Git Bash) |
| make -f verif/scripts/Makefile.questa eclass_regression_l2 | Level-2 regression (WSL2/Git Bash) |

# 5.  Environment Variables

| Variable | Purpose | Example Value |
| --- | --- | --- |
| QUESTA_HOME | QuestaSim install root | C:\intelFPGA_pro\23.1\questa_fse |
| RISCV | xpack GCC toolchain root | C:\tools\xpack-riscv-none-elf-14.2.0 |

# 6.  Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| QUESTA_HOME not set / vsim not found | QuestaSim not on PATH and env var absent | Set $env:QUESTA_HOME — see Step 3 |
| Test-Path "$env:QUESTA_HOME\win64\vsim.exe" returns False | Wrong QUESTA_HOME path | Run Get-ChildItem "C:\intelFPGA_pro" -Filter "vsim.exe" -Recurse to find the correct path |
| riscv-none-elf-gcc: not found | RISCV env var not set or wrong path | Set $env:RISCV to xpack root; verify bin\ subfolder contains gcc.exe |
| No rule to make target gpio_l1 | Running make from wrong directory | Run from repo root: make -C sw gpio_l1 |
| Hex file not found: sw/tests/gpio_test_l2.hex | Firmware not built | Run make -C sw or python scripts/build_sw.py --test gpio_l2 |
| ** Error: vlog-2902 — file not found | eclass_sim.py run from wrong directory | cd to repo root (soc1.1/) before running eclass_sim.py |
| ** Error: vopt-2912 — port not found | Vendor IP not patched for E-class L1 | See README_VENDOR_CHANGES.md — prim_subreg.sv and prim_subreg_ext.sv need ds port |
| tb_level.svh: No such file or directory | Missing +incdir in manual vlog call | eclass_sim.py adds +incdir+<repo_root> automatically; for manual compile add +incdir+. |
| Test times out, no TOHOST written | Wrong define level (L2 test without +define+LEVEL2) | Use eclass_sim.py which sets defines automatically; check hex file matches test name |
| ** Error: (vsim-19) — work library missing | Build directory corrupted or incomplete | Run python scripts/eclass_sim.py <test> --clean |
| missing separator. Stop. (Makefile) | Tabs replaced with spaces in Makefile recipes | Re-clone repo; do not open Makefile in editors that convert tabs to spaces |
| [TB] FAIL in sim.log | Firmware test assertion failed | Read sim.log for the specific FAIL message; upper 16 bits of TOHOST = group, lower = sub-error |
| python-docx not found | Missing pip package (doc generation only) | pip install python-docx — not required for simulation |

# 7.  Next Steps

Once the first simulation passes, use these resources to learn more:

| Document | Read for |
| --- | --- |
| README_ECLASS.md | SoC architecture, address maps, register descriptions |
| README_SIM.md | Detailed simulation flow, suppressed warnings, build artifacts |
| README_MAKEFILE.md | All Makefile targets and options |
| docs/OpenSoC_Tier1_SWI.docx | Register bit-field reference, hardware programming guide |
| docs/OpenSoC_Tier1_FileInventory.docx | Every source file with purpose description |

# 8.  Document Revision History

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | April 2026 | OpenSoC / IIITDM | Initial release. Complete 10-step installation guide for Windows 10/11 and WSL2 Ubuntu. Covers QuestaSim, xpack GCC, Python, firmware build, first simulation, regression, and troubleshooting table. |