OpenSoC Tier-1

Software Interface Document (SWI)

Hardware Programming Guide

IIITDM Chennai  ·  OpenSoC Educational SoC Programme  ·  Rev 1.0  ·  April 2026

# 1.  Introduction

This document is the primary reference for software engineers writing bare-metal firmware, board support packages (BSP), or device drivers targeting the OpenSoC Tier-1 SoC.  It describes every memory region, every peripheral register, and every register bit implemented in the current RTL.  The companion Hardware Programming Guide (Section 6) provides step-by-step code examples for common tasks.

## 1.1  Processor Core

The CPU is an in-order RV32IM instruction-set simulator (rv32i_iss.sv) connected to the rest of the SoC over two independent AXI4 master ports:

IMEM port — instruction fetch (read-only, 32-bit words).

DMEM port — data load/store (read/write, byte-enable strobes).

The ISS implements the full RV32I base integer ISA plus the M-extension (integer multiply and divide).  CSR instructions (CSRRW, CSRRS, etc.) cause the core to halt (halt_o=1); they are not implemented.  ECALL and EBREAK also halt the core.  The reset PC is 0x0001_0000 (Boot ROM base).

## 1.2  Scope

This document covers the peripherals implemented in t1_periph_ss_32.sv (UART, GPIO, SPI master) and the on-chip memories (Boot ROM, SRAM).  The PLIC and CLINT are not implemented in this tier; the timer_irq signal is a free-running pulse from the peripheral subsystem and ext_irq is permanently deasserted.

NOTE:  All register addresses are AXI4 byte addresses.  The CPU generates naturally-aligned 32-bit accesses; sub-word accesses to peripheral registers are not supported and produce undefined results.

# 2.  Address Map

The crossbar (t1_xbar_32.sv) decodes the AXI address and routes transactions to one of three slaves.  Any address not matching the Boot ROM or SRAM windows is routed to the Peripheral subsystem.

| Start Address | End Address | Size | Region | Access | Description |
| --- | --- | --- | --- | --- | --- |
| 0x0001_0000 | 0x0001_03FF | 1 KB | Boot ROM | R | Reset vector; first 3 words are the jump-to-SRAM bootstrap sequence (lui t0 / lui sp / jalr x0). Entire 64 KB window is decoded; addresses 0x0001_0400–0x0001_FFFF return NOP (0x0000_0013). |
| 0x8000_0000 | 0x8001_FFFF | 128 KB | SRAM | R/W | Single-cycle read / write-with-response SRAM. Full AXI4 slave (AW+W+B+AR+R). Word-byte-enable write strobes supported. The highest word at 0x8000_0FF0 is used by the test framework as the TOHOST signalling register. |
| 0x9000_0000 | 0x9000_002B | 44 B | Peripheral SS | R/W | UART + GPIO + SPI peripheral registers. Only the 11 defined register words are implemented; all other addresses in the 0x9000_0000–0x901F_FFFF window return 0 on read and are silently ignored on write. |

NOTE:  Writes to the Boot ROM address window (0x0001_0000–0x0001_FFFF) are silently discarded by the crossbar (routed to the Peripheral SS, which accepts and ignores them).  The Boot ROM is read-only.

# 3.  Boot ROM

The Boot ROM (t1_boot_rom_32.sv) is mapped at 0x0001_0000.  It contains three 32-bit instructions that redirect the CPU from the ROM address space to the SRAM where the application binary is pre-loaded by the testbench.

| Word | Address | Encoding | Instruction / Purpose |
| --- | --- | --- | --- |
| 0 | 0x0001_0000 | 0x8000_02B7 | lui t0, 0x80000 — t0 = 0x8000_0000 (SRAM base address) |
| 1 | 0x0001_0004 | 0x8001_0137 | lui sp, 0x80010 — sp = 0x8001_0000 (SRAM top / stack origin) |
| 2 | 0x0001_0008 | 0x0002_80E7 | jalr x0, 0(t0) — unconditional jump to SRAM _start; discards return address |
| 3+ | 0x0001_000C+ | 0x0000_0013 | addi x0, x0, 0 — NOP (filler; never reached in normal flow) |

NOTE:  Software must never jump into the Boot ROM range after the initial boot sequence.  The jalr at word 2 is a one-way jump; there is no return path.  The stack pointer is also initialised here as a safety measure; crt0.S re-initialises it defensively.

# 4.  SRAM

The SRAM (t1_sram_top_32.sv) provides 128 KB of read/write storage mapped at 0x8000_0000.  It is the only writable memory on the SoC and holds the application stack, .data, .bss, and heap.

Base address:  0x8000_0000

End address:   0x8001_FFFF  (128 KB)

Word depth:    32 768 × 32-bit words

Access width:  32 bits with individual byte-enable strobes (wstrb[3:0])

Read latency:  2 AXI cycles  (AR accepted → R valid)

Write latency: 3 AXI cycles  (AW accepted → W accepted → B valid)

## 4.1  TOHOST Convention

The test framework reserves the highest 32-bit word of the SRAM as a TOHOST signalling register.  Software writes to this address to signal pass/fail to the testbench:

#define TOHOST  (*(volatile uint32_t *)0x80000FF0u)

TOHOST = 1u;      /* PASS */

TOHOST = 0xXXXXu; /* FAIL — encode error code in bits [15:0] */

NOTE:  The testbench polls TOHOST every clock cycle.  Any non-zero write terminates the simulation.  Value 1 is PASS; any other non-zero value is treated as a FAIL error code.

# 5.  Peripheral Subsystem Register Reference

All peripheral registers are located in the t1_periph_ss_32 block, base address 0x9000_0000.  Each register is 32 bits wide and must be accessed with 32-bit aligned addresses.  Byte or half-word accesses produce undefined results.

## 5.1  Register Summary

| Offset | Address | Name | Access | Reset |
| --- | --- | --- | --- | --- |
| 0x00 | 0x9000_0000 | UART_CTRL | R/W | 0x0001_0364 |
| 0x04 | 0x9000_0004 | UART_STAT | RO | 0x0000_0001 |
| 0x08 | 0x9000_0008 | UART_TX | WO | — |
| 0x0C | 0x9000_000C | UART_RX | RO | 0x0000_0000 |
| 0x10 | 0x9000_0010 | GPIO_OUT | R/W | 0x0000_0000 |
| 0x14 | 0x9000_0014 | GPIO_OE | R/W | 0x0000_0000 |
| 0x18 | 0x9000_0018 | GPIO_IN | RO | (ext.) |
| 0x1C | 0x9000_001C | SPI_CTRL | R/W | 0x0004_0001 |
| 0x20 | 0x9000_0020 | SPI_STAT | RO | 0x0000_0000 |
| 0x24 | 0x9000_0024 | SPI_TX | WO | — |
| 0x28 | 0x9000_0028 | SPI_RX | RO | 0x0000_0000 |

## 5.2  Detailed Register Descriptions

UART_CTRL  —  0x9000_0000  (offset 0x00)  Reset: 0x0001_0364

UART control register.  Programs the baud-rate divisor and frame format.  Reset value 0x0001_0364 = baud_div 868, uart_en=1, no parity (115 200 baud at 100 MHz).

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:19] | RESERVED | RO | Reserved. Write 0; reads return 0. |
| [18] | PARITY_EVEN | R/W | 1 = even parity (total 1-bits in frame are even). 0 = odd parity. Only meaningful when PARITY_EN=1. |
| [17] | PARITY_EN | R/W | 1 = insert and check one parity bit per frame (8E1 / 8O1). 0 = no parity bit (8N1). |
| [16] | UART_EN | R/W | 1 = UART transmitter and receiver enabled. 0 = disabled; TX line held high (idle); RX FSM frozen. |
| [15:0] | BAUD_DIV | R/W | Bit-period length in system-clock cycles. baud_rate = f_clk / BAUD_DIV. Minimum meaningful value: 4. Example: 868 → 115 200 baud at 100 MHz. For fast simulation tests use BAUD_DIV = 10. |

UART_STAT  —  0x9000_0004  (offset 0x04)  Reset: 0x0000_0001

UART status register (read-only).  Error flags (bits [5:3]) are sticky and cleared on any read of UART_STAT.

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:6] | RESERVED | RO | Reserved. Always 0. |
| [5] | PAR_ERR | RO | Parity error (sticky). Set when the received parity bit does not match the expected parity. Cleared on read of UART_STAT. Valid only when PARITY_EN=1. |
| [4] | FRAME_ERR | RO | Framing error (sticky). Set when the stop bit sampled by the RX FSM is 0 instead of 1. Cleared on read of UART_STAT. |
| [3] | RX_OVF | RO | Receive overflow (sticky). Set when a new complete frame arrives before the CPU has read the previous byte from UART_RX. The old byte is overwritten. Cleared on read of UART_STAT or UART_RX. |
| [2] | RX_VALID | RO | Receive data valid. 1 = UART_RX holds an unread received byte. Cleared automatically when the CPU reads UART_RX. |
| [1] | TX_FULL | RO | Transmit buffer full. 1 = the one-entry TX buffer holds a byte awaiting transmission. A write to UART_TX while TX_FULL=1 is silently discarded. |
| [0] | TX_EMPTY | RO | Transmit idle. 1 = TX state machine is idle AND TX buffer is empty. Safe to write the next byte. Cleared immediately when a byte is queued. |

UART_TX  —  0x9000_0008  (offset 0x08)  Reset: —

UART transmit data register (write-only).  Writing queues one byte for serial transmission.  If TX_FULL=1 the write is silently dropped.  Reads return 0x0000_0000.

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:8] | RESERVED | WO | Ignored on write. |
| [7:0] | TX_DATA | WO | Byte to transmit. Written to the one-entry TX buffer. The UART FSM begins serialising it (start + 8 data bits + optional parity + stop) as soon as the line is idle. |

UART_RX  —  0x9000_000C  (offset 0x0C)  Reset: 0x0000_0000

UART receive data register (read-only).  Returns the last fully received byte.  Reading clears RX_VALID and RX_OVF in UART_STAT.

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:8] | RESERVED | RO | Always 0. |
| [7:0] | RX_DATA | RO | Last received byte (zero-extended to 32 bits). Valid only when RX_VALID=1. Reading this register clears RX_VALID (and RX_OVF) automatically. |

GPIO_OUT  —  0x9000_0010  (offset 0x10)  Reset: 0x0000_0000

GPIO output data register.  Each bit directly drives the corresponding gpio_out_o pin when the matching GPIO_OE bit is 1.

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:0] | GPIO_OUT[31:0] | R/W | Output value for GPIO pins 31..0. Bit n sets gpio_out_o[n]. Bit value is latched and held until written again. |

GPIO_OE  —  0x9000_0014  (offset 0x14)  Reset: 0x0000_0000

GPIO output-enable register.  A 1 in bit n enables the corresponding GPIO_OUT bit to drive the output pin.  A 0 makes the pin tri-stated (input mode).

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:0] | GPIO_OE[31:0] | R/W | Output-enable for GPIO pins 31..0. 1 = pin is driven by GPIO_OUT[n]. 0 = pin is in input mode (tri-stated in the pad ring). No pull-ups or pull-downs are implemented in the RTL model. |

GPIO_IN  —  0x9000_0018  (offset 0x18)  Reset: (ext.)

GPIO input data register (read-only).  Returns the current value of the gpio_in_i input bus.  No synchroniser is inserted in the RTL model; it is the SoC integrator's responsibility to synchronise asynchronous inputs before they reach gpio_in_i.

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:0] | GPIO_IN[31:0] | RO | Live sampled value of the 32 GPIO input pins. Read combinatorially from the gpio_in_i port. |

SPI_CTRL  —  0x9000_001C  (offset 0x1C)  Reset: 0x0004_0001

SPI master control register.  Configures clock rate and enables the SPI master.  Reset value: SPI_EN=0, CLK_DIV=4.  Software must set SPI_EN=1 before initiating any transfer.

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:16] | RESERVED | RO | Reserved. Always 0. |
| [15:8] | CLK_DIV | R/W | SPI clock half-period in system-clock cycles. SCK frequency = f_clk / (2 × CLK_DIV). Minimum value: 2. Example: CLK_DIV=4 → SCK = 12.5 MHz at 100 MHz system clock. CLK_DIV=50 → SCK ≈ 1 MHz. |
| [7:1] | RESERVED | RO | Reserved. |
| [0] | SPI_EN | R/W | 1 = SPI master enabled. 0 = disabled; SCK=0, CSB=1, MOSI=0. A write to SPI_TX while SPI_EN=0 is ignored. |

SPI_STAT  —  0x9000_0020  (offset 0x20)  Reset: 0x0000_0000

SPI master status register (read-only).  Polled by software to detect transfer completion.

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:2] | RESERVED | RO | Reserved. Always 0. |
| [1] | SPI_DONE | RO | Transfer done flag. Set to 1 when a transfer completes (one clock after the final SCK falling edge and CSB deassertion). Cleared automatically when the CPU reads SPI_RX. |
| [0] | SPI_BUSY | RO | 1 = a transfer is in progress (CSB asserted, SCK toggling). 0 = SPI master is idle. Software must wait until SPI_BUSY=0 before starting the next transfer. |

SPI_TX  —  0x9000_0024  (offset 0x24)  Reset: —

SPI transmit data register (write-only).  Writing this register while SPI_EN=1 and SPI_BUSY=0 loads the byte and immediately triggers a single-byte SPI transfer.  If SPI_BUSY=1 or SPI_EN=0 the write is silently ignored.  Reads return 0x0000_0000.

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:8] | RESERVED | WO | Ignored on write. |
| [7:0] | TX_DATA | WO | Byte to transmit over SPI, MSB first. On the first SCK rising edge, bit 7 is driven on MOSI. Bits shift left on each subsequent SCK falling edge. |

SPI_RX  —  0x9000_0028  (offset 0x28)  Reset: 0x0000_0000

SPI receive data register (read-only).  Holds the byte clocked in on MISO during the most recent completed transfer.  Reading clears SPI_DONE in SPI_STAT.

| Bits | Field Name | Access | Description |
| --- | --- | --- | --- |
| [31:8] | RESERVED | RO | Always 0. |
| [7:0] | RX_DATA | RO | Received byte (zero-extended). Assembled MSB-first during the SPI transfer: bit 7 is sampled on the first SCK rising edge. Valid only when SPI_DONE=1. Reading clears SPI_DONE. |

# 6.  Hardware Programming Guide

This section provides complete C code examples for the most common software tasks on the OpenSoC Tier-1 SoC.  All examples assume a 100 MHz system clock unless stated otherwise.

## 6.1  Minimal C Header (soc.h)

Include this header in every source file that accesses peripherals:

#ifndef SOC_H

#define SOC_H

#include <stdint.h>

/* ---- Base addresses ------------------------------------------- */

#define PERIPH_BASE   0x90000000u

#define SRAM_BASE     0x80000000u

#define TOHOST        (*(volatile uint32_t *)0x80000FF0u)

/* ---- UART registers ------------------------------------------- */

#define UART_CTRL   (*(volatile uint32_t *)(PERIPH_BASE + 0x00u))

#define UART_STAT   (*(volatile uint32_t *)(PERIPH_BASE + 0x04u))

#define UART_TX     (*(volatile uint32_t *)(PERIPH_BASE + 0x08u))

#define UART_RX     (*(volatile uint32_t *)(PERIPH_BASE + 0x0Cu))

/* UART_CTRL fields */

#define UART_BAUD(x)      ((uint32_t)(x) & 0xFFFFu)

#define UART_EN           (1u << 16)

#define UART_PARITY_EN    (1u << 17)

#define UART_PARITY_EVEN  (1u << 18)

/* UART_STAT fields */

#define UART_TX_EMPTY  (1u << 0)

#define UART_TX_FULL   (1u << 1)

#define UART_RX_VALID  (1u << 2)

#define UART_RX_OVF    (1u << 3)

#define UART_FRAME_ERR (1u << 4)

#define UART_PAR_ERR   (1u << 5)

/* ---- GPIO registers ------------------------------------------- */

#define GPIO_OUT    (*(volatile uint32_t *)(PERIPH_BASE + 0x10u))

#define GPIO_OE     (*(volatile uint32_t *)(PERIPH_BASE + 0x14u))

#define GPIO_IN     (*(volatile uint32_t *)(PERIPH_BASE + 0x18u))

/* ---- SPI registers -------------------------------------------- */

#define SPI_CTRL    (*(volatile uint32_t *)(PERIPH_BASE + 0x1Cu))

#define SPI_STAT    (*(volatile uint32_t *)(PERIPH_BASE + 0x20u))

#define SPI_TX      (*(volatile uint32_t *)(PERIPH_BASE + 0x24u))

#define SPI_RX      (*(volatile uint32_t *)(PERIPH_BASE + 0x28u))

/* SPI_CTRL fields */

#define SPI_EN            (1u << 0)

#define SPI_CLKDIV(x)     ((uint32_t)(x) << 8)

/* SPI_STAT fields */

#define SPI_BUSY          (1u << 0)

#define SPI_DONE          (1u << 1)

#endif /* SOC_H */

## 6.2  Startup Code Flow

The linker script (eclass.ld) places _start at 0x8000_0000.  The Boot ROM jumps to _start via jalr.  The crt0.S startup code must:

1. Initialise the stack pointer to the top of SRAM (0x8002_0000).

2. Zero the .bss section.

3. Copy .data initializers from flash/ROM to RAM (not required in simulation — testbench pre-loads the binary).

4. Call main().

5. On return from main(), write TOHOST=1 (PASS) and loop forever.

Minimal crt0.S:

.section .text.start

.global _start

_start:

lui  sp, %hi(0x80020000)     # stack top = SRAM end

addi sp, sp, %lo(0x80020000)

call zero_bss                # zero .bss

call main

li   t0, 0x80000FF0          # TOHOST address

li   t1, 1

sw   t1, 0(t0)              # write PASS

1:  j    1b                      # spin forever

## 6.3  UART Programming

### 6.3.1  Configuration

Always configure UART_CTRL before transmitting or receiving.  The reset value enables the UART at 115 200 baud (868 at 100 MHz), 8N1.  Change baud rate and parity as needed:

/* 115200 baud, 8N1, 100 MHz system clock */

UART_CTRL = UART_BAUD(868) | UART_EN;

/* 9600 baud, 8E1 (even parity) */

UART_CTRL = UART_BAUD(10417) | UART_EN | UART_PARITY_EN | UART_PARITY_EVEN;

/* Fast simulation baud (10 clock cycles/bit) */

UART_CTRL = UART_BAUD(10) | UART_EN;

### 6.3.2  Transmitting a Byte

Always check TX_EMPTY (or at minimum TX_FULL) before writing UART_TX.  Writing while TX_FULL=1 silently discards the byte.

void uart_putc(uint8_t c) {

/* Wait until TX is idle and buffer is empty */

while ((UART_STAT & UART_TX_EMPTY) == 0u);

UART_TX = (uint32_t)c;

}

void uart_puts(const char *s) {

while (*s) uart_putc((uint8_t)*s++);

}

### 6.3.3  Receiving a Byte

Poll RX_VALID before reading UART_RX.  Reading UART_RX automatically clears RX_VALID (and RX_OVF).  Always check error flags after reception in production code.

uint8_t uart_getc(void) {

uint32_t stat;

/* Spin until a byte arrives */

while ((UART_STAT & UART_RX_VALID) == 0u);

uint8_t data = (uint8_t)(UART_RX & 0xFFu);

/* Check errors (read clears sticky flags) */

stat = UART_STAT;

if (stat & (UART_FRAME_ERR | UART_PAR_ERR | UART_RX_OVF)) {

/* handle error — stat holds the flags */

}

return data;

}

NOTE:  If you need a non-blocking receive (return immediately if no data), check RX_VALID first: if ((UART_STAT & UART_RX_VALID) == 0u) return -1;

### 6.3.4  Baud Rate Formula

BAUD_DIV = round( f_clk / baud_rate )

| Baud Rate | f_clk = 100 MHz → BAUD_DIV | Frame length (8N1) |
| --- | --- | --- |
| 9 600 | 10 417 | 1.04 ms |
| 19 200 | 5 208 | 521 µs |
| 57 600 | 1 736 | 174 µs |
| 115 200 | 868 | 87 µs |
| 1 000 000 | 100 | 10 µs |
| 10 (sim) | 10 | 1.0 µs |

## 6.4  GPIO Programming

GPIO pins are controlled by three registers: GPIO_OE (direction), GPIO_OUT (output value), and GPIO_IN (input snapshot).

/* Configure lower 8 pins as outputs, upper 24 as inputs */

GPIO_OE  = 0x000000FFu;

GPIO_OUT = 0x00000000u;   /* all outputs initially low */

/* Set pin 3 high, clear pin 0 low */

GPIO_OUT |=  (1u << 3);

GPIO_OUT &= ~(1u << 0);

/* Read all 32 input pins */

uint32_t inputs = GPIO_IN;

/* Test if pin 7 is high */

if (GPIO_IN & (1u << 7)) { /* pin 7 is high */ }

NOTE:  GPIO_OE and GPIO_OUT are read/write registers; you can read them back to check the configured value.  GPIO_IN is read-only and always returns the live external pin state regardless of GPIO_OE.

## 6.5  SPI Programming

The SPI master implements Mode 0 (CPOL=0, CPHA=0): SCK idles low; data is sampled on the rising edge of SCK and shifted out on the falling edge; MSB is transmitted first.  Each transfer is exactly 8 bits.

### 6.5.1  SPI Clock Rate

SCK frequency = f_clk / (2 × CLK_DIV)

| CLK_DIV | SCK @ 100 MHz | Cycles per byte |
| --- | --- | --- |
| 2 | 25.0 MHz | 32 |
| 4 | 12.5 MHz | 64 |
| 8 | 6.25 MHz | 128 |
| 50 | 1.00 MHz | 800 |
| 100 | 500 kHz | 1600 |

### 6.5.2  SPI Transfer Sequence

The SPI hardware handles CSB, SCK, and MOSI automatically once the CPU writes SPI_TX.  Software only needs to poll for completion:

/* 1. Configure: enable SPI, set clock divider */

SPI_CTRL = SPI_EN | SPI_CLKDIV(4u);   /* 12.5 MHz @ 100 MHz */

/* 2. Wait for idle (should be immediate after reset) */

while (SPI_STAT & SPI_BUSY);

/* 3. Start transfer — write TX byte */

SPI_TX = 0xA5u;       /* triggers transfer immediately */

/* 4. Wait for transfer to complete */

while (!(SPI_STAT & SPI_DONE));

/* 5. Read received byte (clears SPI_DONE) */

uint8_t rx = (uint8_t)(SPI_RX & 0xFFu);

### 6.5.3  Multi-byte Transfer Helper

For transfers larger than one byte, keep CSB asserted between bytes by writing SPI_TX again immediately after SPI_DONE — the hardware will start the next byte within one clock cycle.

NOTE:  The current hardware deasserts CSB after each byte (one-byte frame). For devices requiring multi-byte frames with CSB held continuously, the SPI_CTRL register would need a CONT (continuous) mode bit — not yet implemented.  Each SPI_TX write results in a separate CSB-asserted frame.

uint8_t spi_xfer(uint8_t out) {

while (SPI_STAT & SPI_BUSY);   /* ensure idle */

SPI_TX = (uint32_t)out;

while (!(SPI_STAT & SPI_DONE));

return (uint8_t)(SPI_RX & 0xFFu);

}

/* Example: read device register 0x0F */

uint8_t reg_val;

spi_xfer(0x8Fu);         /* send read command (bit 7 = R) */

reg_val = spi_xfer(0x00u);  /* clock in the response byte */

## 6.6  Timer Interrupt

The peripheral subsystem exports a free-running timer interrupt (timer_irq_o) that fires once every 65 536 system-clock cycles.  At 100 MHz this is approximately 655 µs per pulse.

The timer is not memory-mapped; it cannot be configured, read, or disabled by software.  The interrupt signal is available to the PLIC (not implemented in this tier) or can be connected to an external interrupt controller by the SoC integrator.

The external interrupt (ext_irq_o) is permanently deasserted (0) in this design.

# 7.  Linker Script Reference

The standard linker script (eclass.ld) defines:

ENTRY(_start)

MEMORY {

sram  (rwx) : ORIGIN = 0x80000000, LENGTH = 128K

}

SECTIONS {

.text   : { *(.text.start) *(.text*) }  > sram

.rodata : { *(.rodata*)               }  > sram

.data   : { *(.data*)                 }  > sram

.bss    : { *(.bss*) *(COMMON)        }  > sram

. = ALIGN(4);

_heap_start = .;

}

The stack grows downward from 0x8002_0000 (top of SRAM).  Ensure your application does not overflow the heap into the stack.  A 128 KB SRAM gives a combined code+data+stack budget of 128 KB.

# 8.  Build Tool Flow

The RISC-V GCC toolchain (riscv32-unknown-elf-gcc) is used to compile all C tests.

## 8.1  Compiler Flags

| Flag | Purpose |
| --- | --- |
| -march=rv32im | Target RV32IM ISA (base integer + multiply/divide). |
| -mabi=ilp32 | Integer LP32 ABI (all pointers and int are 32-bit). |
| -Os | Optimise for code size (important for 128 KB SRAM). |
| -g | Include DWARF debug info in ELF (for waveform analysis). |
| -nostdlib | Do not link glibc (no OS support). |
| -nostartfiles | Do not include default crt0 (use our own crt0.S). |
| -ffreestanding | Freestanding environment (no hosted C library assumptions). |
| -Wall -Wno-unused-function | Enable warnings; suppress unused-function noise from inline helpers. |

## 8.2  Build and Simulate

The recommended flow uses sw/Makefile for firmware and scripts/eclass_sim.py for simulation.  eclass_sim.py automatically triggers make -C sw if the hex file is missing or its source is newer than the hex.

| Make Target | Action |
| --- | --- |
| make -C sw gpio_l1 | Build L1 GPIO firmware → sw/tests/gpio_test_l1.hex |
| make -C sw gpio_l2 | Build L2 GPIO firmware → sw/tests/gpio_test_l2.hex |
| make -C sw uart_l1 | Build L1 UART firmware → sw/tests/uart_test_l1.hex |
| make -C sw uart_l2 | Build L2 UART firmware → sw/tests/uart_test_l2.hex |
| make -C sw | Build all 4 firmware hex files |
| python scripts/eclass_sim.py gpio_l1 | Compile + simulate L1 GPIO test (auto-builds firmware) |
| python scripts/eclass_sim.py gpio_l2 | Compile + simulate L2 GPIO test |
| python scripts/eclass_sim.py uart_l1 | Compile + simulate L1 UART loopback test |
| python scripts/eclass_sim.py uart_l2 | Compile + simulate L2 UART loopback test |
| python scripts/eclass_sim.py cov_l2 --coverage | Run L2 coverage test and generate HTML report |
| python scripts/eclass_sim.py uart_l2 --gui --waves | Open QuestaSim GUI with waveforms |
| python scripts/eclass_sim.py gpio_l2 --clean | Wipe build directory before simulation |

# 9.  Common Software Errors and Diagnostics

| Symptom | Cause / Fix |
| --- | --- |
| TOHOST never written / timeout | The CPU never reached the TOHOST write. Check: (a) ELF loads at 0x8000_0000 — confirm hex is loaded into u_sram.mem in testbench; (b) crt0.S _start is at 0x8000_0000; (c) main() is not returning early due to a missing return statement or bad pointer. |
| UART TX byte silently discarded | TX_FULL=1 when uart_putc() writes. Always poll TX_EMPTY before writing UART_TX. Back-to-back writes without polling will lose bytes. |
| UART RX_VALID never set | Check: (a) UART_EN=1 in UART_CTRL; (b) baud_div matches testbench echo loop; (c) testbench echo loop is active. If parity is mismatched, the frame error flag will be set and rx_valid may still assert. |
| SPI transfer never completes (SPI_DONE=0) | Check: (a) SPI_EN=1 in SPI_CTRL before writing SPI_TX; (b) CLK_DIV >= 2; (c) SPI_BUSY was 0 before the SPI_TX write. If SPI_EN=0, writes to SPI_TX are silently ignored. |
| SPI received byte is 0x00 | MISO (spi_sd_i) is not being driven. Verify the testbench SPI slave is running and driving spi_sd_i before the first SCK rising edge. |
| GPIO_IN always reads 0 | gpio_in_i is not driven by the testbench. In t1_gpio_cpu_tb, the expected value must match GPIO_IN_EXPECTED in gpio_test.c. |
| ISS halts immediately (halt_o=1) | The CPU encountered an illegal opcode (0x00000000), ECALL, EBREAK, or a CSR instruction. Most likely cause: the .hex file was not loaded into SRAM, so all reads return 0x00000000 which decodes as illegal. |

# 10.  Document Revision History

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | April 2026 | OpenSoC / IIITDM | Initial release. UART + GPIO + SPI master registers. Full register bit descriptions. Hardware Programming Guide with C code examples. |
| 1.1 | April 2026 | OpenSoC / IIITDM | Updated Section 8.2: new firmware build flow (sw/Makefile + make -C sw) and cross-platform simulation runner (scripts/eclass_sim.py). Added L2 UART and GPIO firmware (uart_test_l2.c, gpio_test_l2.c). SPI/I2C ports now conditionally compiled via ifdef LEVEL2 in RTL and testbenches. |