# OpenSoC Tier-1 SoC
Detailed Design Document

Version 1.3  ·  April 2026
QuestaSim 2024.1 verified  ·  CVA6 v5.x  ·  OpenTitan earlgrey_1.0

Document Sections: 1. Introduction  |  2. SoC Architecture  |  3. CVA6 CPU Subsystem  |  4. Memory Subsystem  |  5. AXI Interconnect  |  6. Peripheral Subsystem  |  7. Interrupt & Timer Architecture  |  8. Debug & JTAG  |  9. Clock & Reset  |  10. Simulation & Verification  |  11. RTL Module Descriptions  |  12. Signal Interfaces  |  13. Timing Diagrams  |  14. Power & Area  |  15. Known Issues

# 1. Introduction

The OpenSoC Tier-1 SoC is a minimal, open-source, single-core RISC-V System-on-Chip designed for educational VLSI/SoC design at IIITDM. The design targets SKY130 tape-out via the Efabless Open MPW shuttle and FPGA prototype on the Digilent Arty A7-100T.

This Detailed Design Document describes every RTL module, their interfaces, internal microarchitecture, parameter settings, and their inter-connections. It records the full simulation bring-up journey: the errors encountered porting the design to QuestaSim 2024.1 on Windows and all fixes applied.

## 1.1 Design Goals

Minimal gate count: target < 400 kGE on SKY130

Fully open-source IP stack: no proprietary blocks

Tape-out ready: OpenLane 2 / OpenROAD flow on SKY130 PDK

FPGA prototype target: Digilent Arty A7-100T

Boot to bare-metal application in < 100 ms at 50 MHz

All verification open-source: Verilator, UVM, SymbiYosys, Cocotb

QuestaSim 2024.1 simulation on Windows (achieved March 2026, 0 errors)

## 1.2 Document Scope

This document covers only the Tier-1 implementation. Tier-2 and Tier-3 features (multi-core, DRAM, accelerators, full OpenTitan RoT) are deferred and described in the baseline platform specification OpenSoC_Platform_Specification_v1.0.docx.

# 2. SoC Architecture

## 2.1 Block Diagram

The following shows the top-level hierarchy of the OpenSoC Tier-1 SoC:

t1_soc_top
├── t1_clkrst              — clock/reset generation and synchronisation
├── cva6_wrapper           — CVA6 RV64GC core (v5.x, 6-stage in-order)
│     └── cva6             — full CVA6 pipeline + caches + MMU + FPU
├── t1_xbar                — 1×3 AXI4 crossbar (64-bit, PULP axi_xbar)
│     ├── Slave 0: Boot ROM port  — 0x0001_0000 – 0x0001_FFFF
│     ├── Slave 1: SRAM port      — 0x8000_0000 – 0x800F_FFFF
│     └── Slave 2: Peripheral port— 0x9000_0000 – 0x901F_FFFF
├── t1_boot_rom            — 64-entry × 8-byte synthesised ROM
├── t1_sram_top            — 128 KB on-chip SRAM
│     └── axi_sram_ctrl   — AXI4 → SRAM single-port adapter
└── t1_periph_ss           — Peripheral subsystem
      ├── (AXI-Lite → TL-UL bridge)
      ├── uart             — OpenTitan UART (rv_uart)
      ├── gpio             — OpenTitan GPIO
      ├── rv_timer         — OpenTitan 2-channel 64-bit timer
      ├── aon_timer        — OpenTitan watchdog
      ├── spi_host         — OpenTitan SPI Host
      ├── i2c              — OpenTitan I2C
      └── rv_plic          — RISC-V PLIC (64 sources, 2 targets)

## 2.2 Top-Level Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| AxiAddrWidth | 64 | AXI address bus width (matches CVA6ConfigAxiAddrWidth=64) |
| AxiDataWidth | 64 | AXI data bus width |
| AxiIdWidth | 4 | AXI ID width from CVA6 |
| AxiUserWidth | 1 | AXI user sideband width |
| SramNumWords | 16384 | SRAM depth in 64-bit words (= 128 KB) |
| RomDepth | 64 | Boot ROM entries (= 512 B at 8 bytes/entry) |

## 2.3 Top-Level Ports

| Port | Direction | Width | Description |
| --- | --- | --- | --- |
| clk_i | input | 1 | System clock (100 MHz for simulation) |
| rst_ni | input | 1 | Active-low synchronous reset |
| jtag_tck_i | input | 1 | JTAG test clock |
| jtag_tms_i | input | 1 | JTAG test mode select |
| jtag_trst_ni | input | 1 | JTAG async reset (active-low) |
| jtag_tdi_i | input | 1 | JTAG data in |
| jtag_tdo_o | output | 1 | JTAG data out |
| jtag_tdo_oe_o | output | 1 | JTAG TDO output enable |
| uart_rx_i | input | 1 | UART receive data (idle = 1) |
| uart_tx_o | output | 1 | UART transmit data (idle = 1) |
| gpio_in_i | input | 32 | GPIO input values |
| gpio_out_o | output | 32 | GPIO output values |
| gpio_oe_o | output | 32 | GPIO output-enable (1 = driving) |
| spi_sck_o | output | 1 | SPI clock (idle = 0, CPOL=0) |
| spi_csb_o | output | 1 | SPI chip-select (idle = 1) |
| spi_sd_i | input | 1 | SPI MISO |
| spi_sd_o | output | 1 | SPI MOSI |
| spi_sd_oe_o | output | 1 | SPI SD output enable |

# 3. CVA6 CPU Subsystem

## 3.1 Module: cva6_wrapper

File: rtl/cpu/cva6_wrapper.sv
The wrapper configures CVA6 for Tier-1 using the v5.x API and connects it to the AXI crossbar.

| Parameter / Signal | Value / Connection | Notes |
| --- | --- | --- |
| Config package | cv64a6_imafdc_sv39_config_pkg::cva6_cfg | Provides all CVA6 config parameters |
| boot_addr_i | 64'h0000_0000_0001_0000 | Boot ROM base address |
| hart_id_i | 64'h0 | Single hart |
| irq_i[1] | plic_irq | Machine External Interrupt from PLIC |
| irq_i[0] | 1'b0 | Supervisor external (unused Tier-1) |
| ipi_i | 1'b0 | No IPI in Tier-1 |
| time_irq_i | 1'b0 | CLINT timer (planned Tier-2) |
| debug_req_i | 1'b0 | Debug module (planned Tier-2) |
| noc_req_o | → t1_xbar master port | CVA6 v5.x AXI request bus |
| noc_resp_i | ← t1_xbar master port | CVA6 v5.x AXI response bus |

## 3.2 CVA6 Pipeline Stages

| Stage | Function | Key units |
| --- | --- | --- |
| Fetch (IF) | PC generation, I-cache access, branch prediction | BTB-16, BHT-64, RAS-4, I-cache 16 KB 4-way |
| Decode (ID) | Instruction decode, compressed expansion | Compressed decoder, macro decoder |
| Issue (IS) | Operand read, hazard check, issue to FU | Scoreboard, register file (FF-based), bypass network |
| Execute (EX) | ALU, mult/div, branch, FPU, LSU | ALU, multiplier (1-cycle MUL), serdiv, FPU (fpnew), branch unit |
| Memory (MEM) | D-cache access, TLB, MMU | D-cache 16 KB 4-way, D-TLB 16-entry, PTW, PMP |
| Commit (CM) | In-order commit, CSR update, exception/trap | Commit stage, CSR regfile, perf counters |

## 3.3 Memory Management Unit (Sv39)

CVA6 implements the RISC-V Sv39 virtual memory scheme: 39-bit virtual address, 3-level page table, 16-entry I-TLB, 16-entry D-TLB, hardware page table walker (PTW) that issues AXI4 loads to walk the page table. M-mode accesses bypass the MMU. PMP provides 8 physical memory protection regions.

# 4. Memory Subsystem

## 4.1 Boot ROM — t1_boot_rom

File: rtl/memory/t1_boot_rom.sv
Implements a small synthesisable ROM as a case statement on the AXI address. The ROM contents are compiled into the case statement by 03_stitch_ips.ps1 from a .hex image. In simulation the ROM is empty (all zeros); the CVA6 will attempt to fetch from 0x0001_0000 but receive zero-words.

| Parameter | Default | Description |
| --- | --- | --- |
| AxiAddrWidth | 64 | Must match t1_xbar |
| AxiDataWidth | 64 | 64-bit words |
| RomDepth | 64 | Number of 64-bit entries (= 512 bytes) |

AXI behaviour: 1-cycle read latency. Writes return SLVERR (AXI error). Address bits [8:3] select the ROM entry (6 bits → 64 entries).

## 4.2 On-Chip SRAM — t1_sram_top + axi_sram_ctrl

Files: rtl/memory/t1_sram_top.sv, rtl/memory/axi_sram_ctrl.sv
t1_sram_top instantiates tc_sram (a technology-cell SRAM wrapper) and wraps it with the AXI4 adapter axi_sram_ctrl.

| Parameter | Default | Description |
| --- | --- | --- |
| AxiAddrWidth | 64 | Must match t1_xbar |
| AxiDataWidth | 64 | 64-bit data bus |
| SramNumWords | 16384 | SRAM depth in words (128 KB @ 8 bytes/word) |

axi_sram_ctrl maps AXI4 read/write bursts to the single-port SRAM. Byte enables are extracted from AXI wstrb. Read latency is 2 cycles (registered). Burst support: INCR and WRAP; FIXED is supported trivially (same address). The SRAM backend is tc_sram, which on FPGA maps to Xilinx BRAM18; on SKY130 it tiles sky130_sram_2kbyte_1rw1r_32x512_8 macros.

# 5. AXI Interconnect — t1_xbar

File: rtl/interconnect/t1_xbar.sv
Wraps the PULP Platform axi_xbar module configured as 1 master × 3 slaves.

## 5.1 Parameters

| Parameter | Value | Description |
| --- | --- | --- |
| AxiAddrWidth | 64 | AXI address width (corrected from 40 in v1.0) |
| AxiDataWidth | 64 | AXI data width |
| AxiIdWidth | 4 | Master ID width |
| AxiUserWidth | 1 | User sideband |
| NoMasterPorts | 1 | CVA6 is the sole AXI master |
| NoSlavePorts | 3 | Boot ROM, SRAM, Peripheral |

## 5.2 Address Map Rules

| Slave index | Slave | Start address | End address |
| --- | --- | --- | --- |
| 0 | Boot ROM | 0x0000_0000_0001_0000 | 0x0000_0000_0001_FFFF |
| 1 | On-chip SRAM | 0x0000_0000_8000_0000 | 0x0000_0000_800F_FFFF |
| 2 | Peripheral subsys. | 0x0000_0000_9000_0000 | 0x0000_0000_901F_FFFF |

NOTE: All address literals use 64'h format. The v1.0 spec used 40'h format causing an addr_decode_dync assume() failure at time 0 in QuestaSim — corrected in v1.1.

## 5.3 Crossbar Internals (axi_xbar)

The PULP axi_xbar is a fully registered, non-coherent AXI4 crossbar. Each slave port has an axi_demux that routes incoming transactions from the master based on address decoding (addr_decode). Each master port has an axi_mux that arbitrates between slave-to-master responses. ID prepending (axi_id_prepend) adds slave-index bits to the AXI ID for routing responses back to the correct slave. ATOPs (atomic operations) are handled by axi_atop_filter. A DECERR slave (axi_err_slv) responds to unmapped addresses.

# 6. Peripheral Subsystem — t1_periph_ss

File: rtl/peripherals/t1_periph_ss.sv
Bridges AXI4-Lite from the crossbar to the TileLink Uncached Lightweight (TL-UL) bus used by all OpenTitan peripheral IP. A 3-bit device-select (dev_sel) demultiplexes TL-UL transactions to the appropriate peripheral based on address bits [14:12]. Each peripheral's reg_top receives a separate tl_h2d channel.

## 6.1 Device Select Encoding

| dev_sel | Peripheral | Offset from 0x9000_0000 |
| --- | --- | --- |
| 0 | UART | 0x0000 |
| 1 | GPIO | 0x1000 |
| 2 | RV_TIMER | 0x2000 |
| 3 | AON_TIMER | 0x3000 |
| 4 | SPI_HOST | 0x4000 |
| 5 | I2C | 0x5000 |
| 6 | RV_PLIC | 0x6000 |
| 7 | None | — |

## 6.2 Multiple-Driver Fix (v1.1)

The original RTL had two assign statements both driving dev_tl_h2d[i] in the g_dev_mux generate block — one for the full struct and one overriding .a_valid. This is a multiple-driver error (vopt-7045). Fixed by replacing both assigns with an always_comb block where the second assignment takes priority:

// Before (vopt-7045: multiple drivers):
assign dev_tl_h2d[i]         = tl_out;
assign dev_tl_h2d[i].a_valid = tl_out.a_valid && (dev_sel == 3'(i));

// After (always_comb: second line takes priority):
always_comb begin
  dev_tl_h2d[i]         = tl_out;
  dev_tl_h2d[i].a_valid = tl_out.a_valid && (dev_sel == 3'(i));
end

## 6.3 UART (rv_uart) — Detailed

The OpenTitan UART is a full-featured UART with NCO-based baud generation, independent TX/RX FIFOs, and 8 interrupt sources. The NCO register determines baud rate: NCO = (2^20 × baud) / peri_clk. At 115200 baud with 50 MHz peri_clk: NCO = (1048576 × 115200) / 50,000,000 = 2416. The TX path: software writes to WDATA → TX FIFO (32 deep) → uart_tx.sv shift register → uart_tx_o. The RX path: uart_rx_i → uart_rx.sv (16× oversampling) → RX FIFO (64 deep) → RDATA read by software.

| Register | Offset | Function |
| --- | --- | --- |
| CTRL | 0x10 | TX/RX enable, NCO baud config, parity, sys_loopback |
| STATUS | 0x14 | txfull, rxfull, txempty, rxidle, txidle |
| RDATA | 0x1C | Read received byte from RX FIFO |
| WDATA | 0x1C | Write byte to TX FIFO |
| FIFO_CTRL | 0x20 | RX/TX watermark levels, FIFO reset |
| INTR_STATE | 0x00 | Pending interrupts (W1C) |
| INTR_ENABLE | 0x04 | Interrupt enable bits |

## 6.4 SPI Host — Detailed

The OpenTitan SPI Host supports Standard/Dual/Quad modes with 4 chip selects. Transactions are programmed via the COMMAND register: direction (TX-only, RX-only, bidir), width (std/dual/quad), segment length (byte count), and CSAAT (chip-select active after transaction). Data is queued in the TX FIFO (256 bytes) and dequeued from the RX FIFO (256 bytes). The SCK clock is derived from peri_clk with a configurable clock-divider ratio in the CONFIGOPTS register. Idle state: spi_csb_o = 1, spi_sck_o = CPOL.

## 6.5 I2C — Detailed

The OpenTitan I2C block supports both controller (master) and target (slave) roles. In Tier-1, controller mode is used. Speed: Standard (100 kHz), Fast (400 kHz), Fast-mode Plus (1 MHz), configurable via TIMING0–TIMING4 registers. The FMT FIFO (64 bytes) holds format byte pairs: {nakok, rcont, start, stop, readb, fbyte}. The ACQ FIFO (64 bytes) holds received data with signal bits. I2C uses open-drain SCL/SDA — external pull-ups are required.

## 6.6 GPIO — Detailed

32 bidirectional GPIO pins. Each pin independently controlled by:
  DATA_OE_O — output enable register (1 = driving)
  DIRECT_OUT — output data register
  DATA_IN — sampled input value
Each pin has a programmable interrupt: rising-edge, falling-edge, both-edge, or level. In simulation reset state: DATA_OE_O = 0 (all inputs), gpio_out_o = 0.

## 6.7 PLIC (rv_plic) — Detailed

The RISC-V Platform-Level Interrupt Controller supports 64 interrupt sources and 2 targets (hart 0 M-mode, hart 0 S-mode). Each source has a configurable priority (0–7, 3-bit). Each target has a priority threshold register; only interrupts with priority > threshold are forwarded. The claim/complete cycle: software reads CC0 to claim the highest-priority pending interrupt (atomically clearing the pending bit), then writes CC0 with the interrupt ID to signal completion (re-enabling the gateway).

| Signal | Connection | Description |
| --- | --- | --- |
| intr_src_i[31:0] | All peripheral interrupt outputs | Raw interrupt lines into PLIC |
| irq_o[0] | CVA6 irq_i[1] (MEI) | Machine External Interrupt to CVA6 |
| tl_i / tl_o | t1_periph_ss TL-UL bus | Register access from software |

# 7. Interrupt & Timer Architecture

## 7.1 Interrupt Flow

Peripheral (UART/SPI/I2C/GPIO/timer) → intr_src[N]
  ↓
rv_plic (priority arbiter, threshold filter)
  ↓  irq_o[0]
cva6_wrapper irq_i[1]  (Machine External Interrupt = MEI)
  ↓  (inside CVA6: mip.MEIP set)
CVA6 pipeline: exception handler / trap

## 7.2 rv_timer (Dual 64-bit Timer)

Two independent 64-bit compare timers with 12-bit prescaler. Each timer fires an interrupt when TIMER_V >= TIMER_CTRL.COMPARE. One-shot or periodic (software re-arms). Prescaler divides peri_clk: effective tick rate = peri_clk / (prescaler + 1).

## 7.3 AON Timer (Watchdog)

Always-on timer provides watchdog functionality with two stages: Bark: first timeout fires an interrupt (PLIC source 55) — software can recover. Bite: second timeout asserts SoC reset — hardware recovery. The watchdog counter must be pet (WKUP_CTRL.BITE_WDOG_EN write) before expiry. REGWEN register locks the configuration to prevent accidental disabling.

# 8. Debug & JTAG Architecture

## 8.1 Debug Module (dm_top)

The RISC-V debug module (dm_top) implements the RISC-V External Debug Support Specification 1.0. It is accessed via JTAG through the DTM (Debug Transport Module). The DM connects to the SoC via:
  ndmreset_o → connected to CPU reset (not full SoC reset)
  debug_req_o → CVA6 debug_req_i (halt request)
In Tier-1, the SBA (System Bus Access) master port is not yet connected to t1_xbar (planned for Tier-2); DM can only access CPU state via abstract commands.

## 8.2 JTAG TAP (dmi_jtag_tap)

Standard IEEE 1149.1 JTAG TAP with DTM-specific IR instructions:
  BYPASS (0x1F): standard JTAG bypass
  IDCODE (0x01): reads 0x249511C3 — OpenSoC Tier-1 ID
  DTM_CONTROL_STATUS (0x10): DMI address width, DM version
  DMI_ACCESS (0x11): read/write Debug Module Interface register

## 8.3 Clock Domain Crossing in Debug Path

The only asynchronous clock domain crossing in Tier-1 is between jtag_clk (TCK) and cpu_clk. This is handled by dmi_cdc.sv using a request/acknowledge handshake with 2-flip-flop synchronisers on each crossing. No other CDC paths exist in Tier-1 (cpu_clk and peri_clk are the same or synchronously related).

# 9. Clock & Reset Architecture

## 9.1 t1_clkrst Module

File: rtl/top/t1_clkrst.sv
Provides:
  1. PLL wrapper stub (replaced by real MMCM/PLL primitive for FPGA/ASIC)
  2. Reset synchroniser: async assert, sync deassert (2-FF for each clock domain)
  3. Clock gating cell (tc_clk_gating — technology cell, FPGA-safe)
In simulation, clk_i passes through directly with no PLL.

## 9.2 Reset Domains

| Domain | Signal | Consumers | De-assert |
| --- | --- | --- | --- |
| Full SoC | rst_ni | All blocks | Sync on clk_i |
| CPU only | cpu_rst_ni | CVA6 pipeline, caches | Sync, 4-cycle delay after rst_ni |
| Peripheral | peri_rst_ni | All OpenTitan peripherals | Sync, 4-cycle delay after rst_ni |
| Debug | dm_rst_ni | Debug module only | Never reset by ndmreset |

# 10. Simulation & Verification

## 10.1 QuestaSim 2024.1 Bring-up Summary

The design was ported to compile and simulate in QuestaSim 2024.1 on Windows in March 2026. Starting from compile errors, all issues were systematically identified and resolved. Final result:

vlog: Errors: 0, Warnings: 165
[TB] Reset released at time 195000
[TB] Simulation completed 2000 cycles — PASS

## 10.2 Major Issues Resolved

| Error | Root Cause | Fix |
| --- | --- | --- |
| vopt-2912: Port 'ds' not found in prim_subreg | Older rv_plic prim_subreg had 8 ports; newer OpenTitan reg_tops expect 9 (added .ds()) | Added ds output port to prim_subreg and prim_subreg_ext |
| vsim-3906: packed/unpacked mismatch on rv_plic prio | rv_plic.sv declared prio as unpacked; rv_plic_target expects packed 2D array | Changed prio to logic [N_SOURCE-1:0][PRIOW-1:0] prio (packed) |
| vopt-2732: FPGA_EN not found in fifo_v3 | CVA6 v5.x passes .FPGA_EN() to every fifo_v3; parameter was missing | Added parameter bit FPGA_EN = 1'b0 to fifo_v3.sv |
| Module not found: shift_reg_gated | rtl/vendor/common_cells/shift_reg.sv instantiated missing shift_reg_gated | Replaced with CVA6 version that uses always_ff/generate (self-contained) |
| Module not found: addr_decode_dync | rtl/vendor/common_cells/addr_decode.sv wrapped missing addr_decode_dync | Replaced with newer version from vendor/common_cells/src/ (contains dync inline) |
| Assume failure at time 0 (addr_decode_dync) | AXI address rules used 40'h literals; CVA6 uses 64-bit AXI → width mismatch | Changed AxiAddrWidth 40→64 in all project RTL; updated address literals to 64'h |
| vopt-7045: multiple drivers on dev_tl_h2d | Two assign statements both driving parts of same struct in t1_periph_ss.sv | Replaced with always_comb block (sequential assignments allowed) |
| vopt-2912: Missing ports in prim_reg_cdc | Stub had wrong parameter names and missing dst_* ports vs aon_timer_reg_top | Rewrote prim_reg_cdc stub with correct DataWidth/ResetVal/DstWrReq params + all dst_ ports |
| vsim-3906: packed/unpacked mismatch on prim_arbiter_tree.data_i | i2c_fifos.sv declares data as unpacked; stub had packed 2D array | Changed stub to logic [DW-1:0] data_i [N] (unpacked) |
| OOP class scope errors in instr_tracer.sv | QuestaSim 2024.1 cannot resolve class-internal scopes in instr_tracer.sv | Replaced with stub in cva6_sim_stubs.sv; real instr_tracer_if.sv kept |
| Boot ROM returning 0x00000000 (vopt dead-code elimination) | t1_boot_rom_32.sv used 'initial begin...end' + 'logic rom[]' array. QuestaSim vopt treats rom[] as undriven (no runtime always block writes it) and optimises to constant 0. CPU ISS halted immediately (S_HALT, halt_o=1). | Replaced initial block with always_comb case(raddr_q) encoding bootstrap instructions as constants: LUI t0/LUI sp/JALR. This forces live combinatorial logic that vopt cannot eliminate. |
| uart_send_recv return-type mismatch (FAIL macro in non-void function) | uart_test.c: FAIL macro used 'return;' (void return) but uart_send_recv returns uint32_t. GCC -Wreturn-mismatch error: cannot use void return in non-void function. | Added FAIL_VAL macro: 'do { TOHOST = code; return 0xFFFFFFFFu; } while(0)'. Replaced 4 FAIL() calls inside uart_send_recv (TX full, TX timeout, RX timeout, error flag check) with FAIL_VAL(). |

## 10.3 Test Results

| Test | File | Result | Coverage |
| --- | --- | --- | --- |
| Smoke test | t1_tb_top.sv | PASS | 2000 cycles, no X |
| Reset stress | t1_reset_tb.sv | PASS | 3 resets, all outputs checked |
| GPIO patterns | t1_gpio_tb.sv | PASS | 70 patterns: walk-1, walk-0, alt |
| UART idle | t1_uart_idle_tb.sv | PASS | 5 RX frames, loopback stable |
| SPI idle | t1_spi_idle_tb.sv | PASS | CSB=1, SCK=0 throughout |
| UART TX/RX E2E | t1_uart_txrx_tb.sv | PASS | 98 bytes: idle, frames, overflow, loopback, back-to-back |
| GPIO CPU test (L1+L2) | t1_gpio_cpu_tb.sv + sw/tests/gpio_test.c | PASS | GPIO_OUT walk-1/walk-0 (32 patterns); GPIO_OE per-pin enable; GPIO_IN readback = 0xA5A5_5A5A. L2 adds random 32-bit patterns. |
| UART CPU test (L1+L2) | t1_uart_cpu_tb.sv + sw/tests/uart_test.c | PASS | UART_CTRL config + readback; TX byte → TB loopback → UART_RX verify; parity/framing/overflow error flags. L2 adds 4 additional frames. |
| SPI CPU test (L1+L2) | t1_spi_cpu_tb.sv + sw/tests/spi_test.c | PASS | SPI_CTRL readback; 4-byte L1 transfers (MOSI 0xA5/5A/00/FF, MISO 0x3C/C3/FF/55); 8-byte L2 adds 0xF0/0F/12/78 → 0x0F/F0/34/56. TB SPI slave logs each transfer and checks MOSI. |

## 10.4 Compilation Commands

# From OpenSoC_Tier1_Windows_Scripts\ (project root):

# Step 1: Compile all RTL (once)
vlog -work work -sv -f rtl/questa_inc.f -f rtl/questa_src.f

# Step 2: Compile and run individual testbench
vlog -work work -sv verif/tb/t1_uart_txrx_tb.sv
vsim -work work -c t1_uart_txrx_tb -do "run -all; quit -f"

# Step 3: Run all tests
sim\run_all_tests.bat

# questa_inc.f contains: +incdir+... paths
# questa_src.f contains: all .sv source files in order

# 11. RTL Module Descriptions

## 11.1 Stub Modules (Simulation Only)

The following stub modules are created in rtl/include/ to allow simulation without the full OpenTitan infrastructure. They provide correct interfaces but minimal or no logic:

| File | Modules | Behaviour |
| --- | --- | --- |
| prim_util_pkg.sv | prim_util_pkg | Provides vbits() function: $clog2(N) with minimum return of 1 |
| prim_subreg_pkg.sv | prim_subreg_pkg | Defines sw_access_e enum: SwAccessRW, RO, WO, W1S, W1C, W0C, RC |
| prim_mubi_pkg.sv | prim_mubi_pkg | MuBi4 type stub; mubi4_t = logic[3:0]; True=1010, False=0101 |
| prim_alert_pkg.sv | prim_alert_pkg | alert_tx_t struct stub |
| prim_ram_1p_pkg.sv | prim_ram_1p_pkg | RAM config struct stub |
| prim_fifo_sync.sv | prim_fifo_sync | Pass-through stub: wready=1, rvalid=0 |
| prim_flop_2sync.sv | prim_flop_2sync | 2-FF synchroniser: two always_ff stages |
| prim_intr_hw.sv | prim_intr_hw | Interrupt HW stub: intr_o = event_intr_i & intr_en_i |
| prim_lc_sync.sv | prim_lc_sync | Replicates lc_en_i to N output copies |
| prim_alert_sender.sv | prim_alert_sender | Drives alert_tx_o.alert_p from alert_req_i.p |
| prim_reg_we_check.sv | prim_reg_we_check | One-hot write-enable checker; err_o = 0 always in sim |
| prim_flop.sv | prim_flop | Parameterised D-FF with async reset |
| prim_edge_detector.sv | prim_edge_detector | Rising-edge and falling-edge pulse outputs |
| prim_reg_cdc.sv | prim_reg_cdc | CDC register stub: src_busy_o=0; all dst_ outputs tied off |
| prim_misc_stubs.sv | 6 stubs | prim_filter_ctr, prim_fifo_sync_cnt, prim_arbiter_tree (unpacked data_i), prim_ram_1p_adv, prim_packer_fifo, prim_flop_en |
| tlul_stubs.sv | 5 stubs | tlul_cmd_intg_chk, tlul_rsp_intg_gen, tlul_adapter_reg, tlul_adapter_sram (all full port sets), tlul_socket_1n |
| cva6_sim_stubs.sv | 2 stubs | instr_tracer (no-op), cva6_rvfi_probes (rvfi_probes_o = '0, packed array ports for wbdata_i and wdata_i) |

# 12. Key Internal Signal Interfaces

## 12.1 AXI4 Bus (ariane_axi::req_t / resp_t)

The CVA6 uses the ariane_axi package for its AXI bus types. These are defined in vendor/cva6/corev_apu/tb/ariane_axi_pkg.sv and carry all five AXI4 channels (AW, W, B, AR, R) as packed structs.

| Channel | Signals | Width | Description |
| --- | --- | --- | --- |
| AW (Write address) | aw_valid, aw_ready, aw_id, aw_addr, aw_len, aw_size, aw_burst, aw_lock, aw_cache, aw_prot, aw_qos, aw_atop, aw_region, aw_user | varies | Write address channel |
| W (Write data) | w_valid, w_ready, w_data, w_strb, w_last, w_user | varies | Write data channel |
| B (Write resp) | b_valid, b_ready, b_id, b_resp, b_user | varies | Write response channel |
| AR (Read address) | ar_valid, ar_ready, ar_id, ar_addr, ar_len, ar_size, ar_burst, ar_lock, ar_cache, ar_prot, ar_qos, ar_region, ar_user | varies | Read address channel |
| R (Read data) | r_valid, r_ready, r_id, r_data, r_resp, r_last, r_user | varies | Read data channel |

## 12.2 TL-UL Bus (tlul_pkg)

The TileLink Uncached Lightweight (TL-UL) bus is used by all OpenTitan peripherals. Key signals in tl_h2d_t (host-to-device):

| Signal | Width | Description |
| --- | --- | --- |
| a_valid | 1 | Request valid |
| a_opcode | 3 | Get(4), PutFullData(0), PutPartialData(1) |
| a_size | 2 | 2^a_size bytes |
| a_address | 32 | Byte address |
| a_data | 32 | Write data |
| a_mask | 4 | Byte enables |
| d_ready | 1 | Device response accepted |

# 13. Key Timing Diagrams

## 13.1 Reset Sequence

clk    ___/‾\_/‾\_/‾\_/‾\_/‾\_/‾\_/‾\_/‾\_/‾\_/‾\_
rst_n  ___/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
       |<- RST_CYCLES=20 ->|<------- simulation ----->
       [TB] Reset released at time 195000 ns

## 13.2 UART RX Frame (8N1 at 115200 baud, 100 MHz clk)

uart_rx  ‾‾‾\_________/‾\_________/‾‾‾‾‾‾‾‾‾\_______‾
         idle  START   b0    b1    b2..b6    b7   STOP  idle
         |<-- 868 clk per bit -->|
         16x oversampled; sampled at centre of each bit period

## 13.3 AXI4 Single Read Transaction

clk      ___/‾\_/‾\_/‾\_/‾\_/‾\_/‾\_
ar_valid ___/‾‾‾‾‾‾‾‾\_________________
ar_ready ______/‾‾‾‾‾‾\________________
ar_addr  ---<ADDR>---X------------------
r_valid  ________________/‾‾‾‾‾‾‾‾\____
r_ready  ____________/‾‾‾‾‾‾‾‾‾‾‾‾\____
r_data   ________________<DATA>---------
         |<--1cy-->|     |<--1cy->|

# 14. Power & Area Estimates (SKY130)

## 14.1 Power Breakdown

| Block | Dynamic (mW) | Leakage (mW) | Total (mW) | % of Total |
| --- | --- | --- | --- | --- |
| CVA6 core pipeline | 38 | 22 | 60 | 26% |
| CVA6 FPU | 12 | 8 | 20 | 9% |
| L1 I-Cache 16 KB | 8 | 10 | 18 | 8% |
| L1 D-Cache 16 KB | 9 | 10 | 19 | 8% |
| AXI4 crossbar | 4 | 2 | 6 | 3% |
| On-chip SRAM 128 KB | 22 | 14 | 36 | 15% |
| Boot ROM | 1 | 1 | 2 | 1% |
| PLIC + timers | 4 | 2 | 6 | 3% |
| Peripheral subsystem | 6 | 3 | 9 | 4% |
| Debug module | 0 | 1 | 1 | <1% |
| Clock tree + PLL | 15 | 0 | 15 | 6% |
| I/O ring (59 pads) | 8 | 4 | 12 | 5% |
| Routing margin (15%) | 19 | 11 | 30 | 13% |
| TOTAL | 146 | 88 | 234 | 100% |

## 14.2 Area Breakdown

| Block | kGE | Cell area (mm²) | % of Core |
| --- | --- | --- | --- |
| CVA6 core pipeline | 280 | 0.403 | 27% |
| L1 I-Cache 16 KB | — | 0.112 | 7% |
| L1 D-Cache 16 KB | — | 0.112 | 7% |
| AXI4 crossbar | 32 | 0.046 | 3% |
| Boot ROM | 4 | 0.006 | <1% |
| On-chip SRAM 128 KB | — | 0.180 | 12% |
| PLIC | 18 | 0.026 | 2% |
| Peripheral subsys. | 28 | 0.040 | 3% |
| Debug module + JTAG | 12 | 0.017 | 1% |
| Clock / PLL | 6 | 0.009 | <1% |
| I/O ring (59 pads) | — | 0.540 | 36% |
| TOTAL | ~380 | ~1.49 mm² | 100% |

## 14.3 Power States

| State | Active blocks | Power | Wakeup latency |
| --- | --- | --- | --- |
| Active (CPU running) | All blocks | ~234 mW | — |
| WFI (CPU halted) | Peripherals, PLIC, SRAM retention | ~68 mW | < 10 µs (IRQ) |
| Sleep (clocks off) | PLL off, SRAM retention, pads hi-Z | ~18 mW | ~1 ms (PLL relock) |
| Deep sleep (SRAM off) | Reset logic + OTP only | ~3 mW | Full reboot |

# 15. Known Issues & Limitations

| Issue | Severity | Description | Workaround / Plan |
| --- | --- | --- | --- |
| Empty Boot ROM (CVA6 SoC — resolved for E-class) | Resolved (E-class) | CVA6 SoC: Boot ROM contains synthesised NOP/jump code. E-class SoC: t1_boot_rom_32.sv now uses always_comb case() with LUI t0, LUI sp, JALR encoded as constants — vopt dead-code elimination fix. CPU successfully fetches and executes bootstrap. | Fixed in April 2026 session. all_comb case() replaces initial block. CPU-driven tests (GPIO/UART/SPI) verified PASS. |
| PMP time-0 assertion | Non-fatal | pmp.sv:89 fires assert(allow_o == 1) before reset deasserts. Simulation still completes PASS. | Accepted as benign init behavior; can be suppressed with -suppress 8386 |
| 165 vlog warnings | Non-fatal | Mostly FPGA_EN parameter overrides (vopt-2732) and addr_i width mismatches. None block simulation. | Can suppress with -suppress 2732 |
| SBA not connected to crossbar | Functional limitation | Debug module system bus access cannot reach SRAM or peripherals. Abstract commands (register access) still work. | Planned for Tier-2: connect dm_top SBA port to t1_xbar master port M2 |
| No real boot firmware | Functional | All simulation tests verify RTL stability, not CPU code execution. | Provide bare-metal firmware image for boot_rom to enable real execution tests |
| No CLINT timer connected | Functional | time_irq_i to CVA6 is tied 0 in t1_soc_top.sv. rv_timer PLIC sources work. | Add CLINT module and connect MTIME/MTIMECMP for Tier-2 |

# 16. Shakti E-class SoC Variant

This section documents the second SoC variant added to OpenSoC Tier-1 in March 2026. The CVA6 (RV64GC) processor is replaced with the Shakti E-class (RV32IMAC, 3-stage in-order, developed by IIT-Madras) to provide a lower-power, smaller-area alternative targeting embedded and IoT applications.

## 16.1 Motivation

Area: E-class is ~2.5× smaller (100–150 kGE vs 280–380 kGE for CVA6 core)

Power: E-class consumes ~3–5× less power (13–22 mW vs 52–98 mW at 100 MHz)

Silicon validation: E-class has been fabricated in 180nm CMOS (IIT-Madras Moushik SoC)

Embedded profile: RV32IMAC is the standard RISC-V embedded ISA (FreeRTOS, Zephyr)

Educational value: 3-stage pipeline is easier to study than CVA6 6-stage

## 16.2 Architecture Changes

| Aspect | CVA6 SoC | E-class SoC |
| --- | --- | --- |
| CPU | cva6_wrapper (RV64GC, 6-stage) | shakti_eclass_wrapper (RV32IMAC, 3-stage) |
| Data width | 64-bit | 32-bit |
| CPU AXI ports | 1 (NOC — combined IMEM+DMEM) | 2 (separate IMEM + DMEM) |
| AXI types | ariane_axi::req_t struct | Flat signals (no package) |
| Crossbar | t1_xbar (PULP axi_xbar 1×3) | t1_xbar_32 (behavioral 2×3) |
| Boot ROM | t1_boot_rom (64-bit, 64 words) | t1_boot_rom_32 (32-bit, 256 words) |
| SRAM | t1_sram_top (64-bit, 16 K words) | t1_sram_top_32 (32-bit, 32 K words) |
| Peripheral SS | t1_periph_ss (full OpenTitan) | t1_periph_ss_32 (behavioral stub) |
| FPU | fpnew (IEEE 754 DP) | None |
| Caches | 16 KB I$ + 32 KB D$ | None (direct AXI) |
| MMU | Sv39 | None |
| Work library | work/ | work_eclass/ |

## 16.3 New RTL Files

| File | Module | Description |
| --- | --- | --- |
| rtl/cpu/shakti_eclass_wrapper.sv | shakti_eclass_wrapper | CPU — AXI4 stub (replace with BSV output) |
| rtl/interconnect/t1_xbar_32.sv | t1_xbar_32 | 32-bit 2-master × 3-slave crossbar |
| rtl/memory/t1_boot_rom_32.sv | t1_boot_rom_32 | 32-bit ROM, 256×32b, RV32I bootstrap |
| rtl/memory/t1_sram_top_32.sv | t1_sram_top_32 | 32-bit SRAM, 128 KB, 1-cycle latency |
| rtl/peripherals/t1_periph_ss_32.sv | t1_periph_ss_32 | Peripheral subsystem for the E-class SoC (32-bit AXI). Implements: UART (TX+RX FSM, configurable baud, 115200 default, error flags); GPIO (32-bit GPIO_OUT/GPIO_OE/GPIO_IN); SPI master (Mode 0 CPOL=0/CPHA=0, MSB-first, 8-bit, CLK_DIV configurable). Register map: UART_CTRL(+0x00), UART_STAT(+0x04), UART_TX(+0x08), UART_RX(+0x0C), GPIO_OUT(+0x10), GPIO_OE(+0x14), GPIO_IN(+0x18), SPI_CTRL(+0x1C), SPI_STAT(+0x20), SPI_TX(+0x24), SPI_RX(+0x28). AXI4-Lite slave, 32-bit data, base 0x9000_0000. |
| rtl/top/t1_soc_top_eclass.sv | t1_soc_top_eclass | E-class SoC top-level |
| rtl/questa_src_eclass.f | — | QuestaSim source file list |
| sw/tests/gpio_test.c spi_test.c uart_test.c | (C tests) | CPU-driven C tests for the E-class SoC. gpio_test.c: GPIO walk patterns + GPIO_IN readback. uart_test.c: UART TX/RX loopback + error flags (uses FAIL_VAL macro for non-void return). spi_test.c: SPI_CTRL config + 8-byte transfer loop with TOHOST PASS/FAIL protocol. All compile with riscv32-unknown-elf-gcc -march=rv32imac -mabi=ilp32. |

## 16.4 Simulation Results

| Testbench | Tests Covered | Cycles | Result |
| --- | --- | --- | --- |
| t1_eclass_tb_top | Reset(3×), Boot, UART, GPIO, SPI | ~5 000 | PASS |
| t1_eclass_smoke_tb | X-free 3000-cycle run, IMEM fetch | 3 000 | PASS |
| t1_eclass_reset_tb | 5× reset sequences, boot after each | ~5 000 | PASS |
| t1_eclass_gpio_tb | 74 GPIO input patterns | ~75 000 | PASS |
| t1_eclass_uart_idle_tb | TX idle 2000 cycles + 8 RX frames | ~90 000 | PASS |
| t1_eclass_spi_idle_tb | SPI 2000-cycle idle (CSB=1, SCK=0) | 2 000 | PASS |
| t1_eclass_uart_txrx_tb | 141 RX bytes, overflow, back-to-back | ~1.3M | PASS |
| TOTAL | 10/10 PASS — 0 FAIL — 0 X events (7 eclass regression + 3 CPU-driven) | > 2 260 000 | PASS |
| t1_gpio_cpu_tb (L1+L2) | GPIO_OUT/OE write, GPIO_IN read (0xA5A5_5A5A); L2 random patterns | ~160 K | PASS |
| t1_uart_cpu_tb (L1+L2) | UART TX/RX loopback, error flags, baud config; L2 extra frames | ~400 K | PASS |
| t1_spi_cpu_tb (L1+L2) | SPI_CTRL readback, 4-xfer L1, 8-xfer L2; TB SPI slave checks MOSI | ~200 K | PASS |
| t1_eclass_gpio_tb | GPIO walk-1/walk-0, L1 (16-bit OpenTitan gpio.sv), boot 0x8000_0000 | 83 370 | PASS — 26 Apr 2026, 0 errors, 0 warnings |
| t1_eclass_gpio_tb | GPIO walk-1/walk-0, L2 (32-bit t1_periph_ss_32), Boot ROM→SRAM 0x0001_0000 | 75 995 | PASS — 26 Apr 2026, 0 errors, 0 warnings |

Simulation run 26 April 2026 — QuestaSim 2024.1 batch mode via scripts/eclass_sim.py

## 16.5 Replacing the CPU Stub

The behavioural stub in shakti_eclass_wrapper.sv should be replaced with the BSV-compiled Verilog output (mkE_Class.v) from the Bluespec toolchain. The replacement procedure is:

1. Install Bluespec compiler (bsc) and the e-class repo: git clone https://gitlab.com/shaktiproject/cores/e-class

2. Run: make -C e-class/src generate_verilog

3. Copy mkE_Class.v to rtl/cpu/mkE_Class.v and add to questa_src_eclass.f

4. In shakti_eclass_wrapper.sv, delete the state-machine stub body and instantiate mkE_Class with port mappings as shown in the comment block.

5. Re-run: make eclass_regression

## 16.6 Document References

OpenSoC_Tier1_Eclass_Simulation_Report.docx — Full verification report (this project)

OpenSoC_Tier1_CPU_Comparison.docx — CVA6 vs E-class performance/power/area comparison

README_ECLASS.md — Integration guide and quick-start

Shakti E-class source: https://gitlab.com/shaktiproject/cores/e-class

Moushik SoC (silicon): https://shakti.org.in/chip.html

# I2C Master Implementation (t1_periph_ss_32)

The I2C master is integrated directly into t1_periph_ss_32.sv following the same baud-counter / phase pattern used by the SPI master.  It is accessed via the peripheral AXI-lite port at base address 0x9000_0000.

## FSM State Encoding

| State | Value | Description |
| --- | --- | --- |
| I2C_IDLE | 4'd0 | Bus idle; waiting for write to I2C_TX register |
| I2C_START | 4'd1 | Generate START condition (SDA 1→0 while SCL=1) |
| I2C_ADDR_SM | 4'd2 | Shift out 8-bit address+RW byte, MSB first |
| I2C_AACK | 4'd3 | Sample ACK/NAK from slave on 9th SCL cycle |
| I2C_WDATA | 4'd4 | Shift out 8-bit data byte (WRITE transfer) |
| I2C_WACK | 4'd5 | Sample ACK/NAK after data byte |
| I2C_RDATA | 4'd6 | Shift in 8-bit data byte from slave (READ transfer) |
| I2C_RACK | 4'd7 | Drive NACK to slave after read byte |
| I2C_STOP | 4'd8 | Generate STOP condition (SDA 0→1 while SCL=1) |
| I2C_DONE_ST | 4'd9 | Assert i2c_done_q for one cycle; return to IDLE |

## Timing Notes

SDA transitions always occur on the falling edge of SCL (i2c_phase_q == 1) to meet I2C setup/hold requirements. START and STOP are the only legal SDA transitions while SCL is high and are generated explicitly in their own states.

## Register Decode

Write decode (6-bit word address from AXI-lite byte address [7:2]):
  6'h0B → I2C_CTRL  (clk_div + enable)
  6'h0C → I2C_ADDR  (slave address + RW bit)
  6'h0D → I2C_TX    (data byte; triggers transfer)

Read decode (same addressing):
  6'h0B → I2C_CTRL
  6'h0C → I2C_ADDR
  6'h0E → I2C_RX    (clears i2c_done_q as side-effect)
  6'h0F → I2C_STAT  {i2c_nak_q, i2c_done_q, i2c_busy_q}

## Testbench Model

The testbench (t1_i2c_cpu_tb.sv) models an I2C slave with:
  • Open-drain bus:  assign i2c_sda_wire = i2c_sda_dut & i2c_sda_slv
  • SCL loopback:    i2c_scl_i = i2c_scl_o  (no clock stretching)
  • Slave FSM:       wait START → receive 8-bit addr → ACK → data → STOP
  • Read data array: i2c_slave_rdata[0]=0xC5, [1]=0x3A (L2 only)

## Test Coverage Matrix

| Scenario | L1 | L2 |
| --- | --- | --- |
| I2C_CTRL readback | ✓ | ✓ |
| Write frame 0 (0xA1) | ✓ | ✓ |
| Write frame 1 (0xA2) | ✓ | ✓ |
| Write frame 2 (0xA3) | — | ✓ |
| Write frame 3 (0xA4) | — | ✓ |
| Read frame 0 (expect 0xC5) | — | ✓ |
| Read frame 1 (expect 0x3A) | — | ✓ |
| NAK detection (TOHOST error) | ✓ | ✓ |
| Timeout detection | ✓ | ✓ |