# OpenSoC Tier-1 Embedded SoC — Technical Specification v1.4

SKY130 tape-out · FPGA prototype · 1× CVA6 RV64GC · Open-source toolchain

# Version History

| Version | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | 2025-03-14 | OpenSoC Team | Initial Tier-1 release |
| 1.1 | March 2026 | OpenSoC Team | QuestaSim 2024.1 simulation verified (0 errors); address map corrected to match implemented RTL (Boot ROM 0x0001_0000, SRAM 0x8000_0000, Periph 0x9000_0000); AXI address width updated 40→64 bits to match CVA6 v5.x; crossbar corrected to 1×3 topology; simulation test results added (Section 15); vendor file change log added (Appendix D). |
| 1.3 | April 2026 | OpenSoC Team | SPI master added to t1_periph_ss_32.sv (SPI_CTRL/STAT/TX/RX registers, Mode 0, 8-bit, configurable CLK_DIV). Boot ROM vopt fix (always_comb case replaces initial block to prevent dead-code elimination). CPU-driven tests PASS: t1_gpio_cpu_tb (L1+L2), t1_uart_cpu_tb (L1+L2), t1_spi_cpu_tb (L1+L2). UART uart_send_recv FAIL_VAL macro fix. SW docs: OpenSoC_Tier1_SWI.docx, OpenSoC_Tier1_FileInventory.docx. soc1.1 snapshot copy created. |
| 1.4 | April 2026 | OpenSoC / IIITDM | E-class SoC primary; GPIO L1 PASS (83 370 cycles) and GPIO L2 PASS (75 995 cycles) added; cover version aligned to v1.4; E-class appendix updated with April 2026 sim results. |

# 1. Introduction & Scope

## 1.1 Purpose

This document specifies the OpenSoC Tier-1 Embedded SoC — the minimal, self-contained implementation of the OpenSoC platform. It is complete for RTL design, verification, and SKY130 tape-out without requiring any Tier-2 or Tier-3 features. Version 1.1 updates the address map, AXI width, and crossbar topology to reflect the implementation as verified in QuestaSim 2024.1 on Windows (March 2026).

## 1.2 Scope Boundaries

| Feature | Tier-1 (this doc) | Deferred to Tier-2/3 |
| --- | --- | --- |
| CPU cores | 1× CVA6 RV64GC @ 50–100 MHz | 4–12× CVA6 + BOOM OoO |
| ISA | RV64GC (G+C) | H (hypervisor), V (vector) |
| L2 cache | None | 512 KB – 4 MB |
| DRAM | None — on-chip SRAM only | DDR4-3200 / HBM2 |
| Interconnect | AXI4 1×3 crossbar (64-bit) | CHI coherency, FlooNoC |
| Peripherals | UART, SPI, I2C, GPIO, Timer, WDT | USB2, Ethernet, PCIe |
| Security | JTAG debug + OTP placeholder | OpenTitan full RoT |
| Accelerators | None | Gemmini, iDMA, RVV |
| Process target | SKY130 / Xilinx Artix-7 FPGA | TSMC 28nm / 12nm |
| OS target | Bare-metal + FreeRTOS | Linux (needs DRAM) |

## 1.3 Key IPs

| IP | Source | License | Version |
| --- | --- | --- | --- |
| CVA6 (Ariane) CPU | OpenHW Group / ETH Zurich | Solderpad 2.0 | v5.0.0 |
| AXI4 crossbar (axi_xbar) | PULP Platform | Solderpad 2.0 | v0.39.0 |
| PLIC (rv_plic) | lowRISC OpenTitan | Apache 2.0 | earlgrey_1.0 |
| UART (rv_uart) | lowRISC OpenTitan | Apache 2.0 | earlgrey_1.0 |
| SPI host | lowRISC OpenTitan | Apache 2.0 | earlgrey_1.0 |
| I2C | lowRISC OpenTitan | Apache 2.0 | earlgrey_1.0 |
| GPIO | lowRISC OpenTitan | Apache 2.0 | earlgrey_1.0 |
| rv_timer / aon_timer | lowRISC OpenTitan | Apache 2.0 | earlgrey_1.0 |
| Debug module (dm_top) | CVA6 / lowRISC | Solderpad 2.0 | v5.0.0 |
| common_cells | PULP Platform | Solderpad 2.0 | v0.39.0 |

# 2. Architecture Overview

## 2.1 Top-Level Description

The Tier-1 SoC is a single-core, non-coherent, on-chip-SRAM-only embedded platform. One CVA6 core connects via a 1-master × 3-slave 64-bit AXI4 crossbar to Boot ROM, on-chip SRAM, and the peripheral subsystem. There is no DRAM, no L2 cache, and no hardware accelerator.

NOTE: v1.1 correction: the implemented crossbar is 1×3 (not 4×6 as stated in v1.0). The additional master ports (debug SBA, reserved DMA) are planned for Tier-2.

## 2.2 Memory Map (Implemented — v1.1 Corrected)

NOTE: The address map below reflects the RTL as synthesised and verified. The v1.0 spec listed Boot ROM at 0x0000_0000 and SRAM at 0x0001_0000; the implementation uses the addresses shown here.

| Slave | Base Address | End Address | Size | Notes |
| --- | --- | --- | --- | --- |
| Boot ROM | 0x0000_0000_0001_0000 | 0x0000_0000_0001_FFFF | 64 KB | Execute-only; 64-entry ROM |
| On-chip SRAM | 0x0000_0000_8000_0000 | 0x0000_0000_800F_FFFF | 1 MB | 128 KB default (SramNumWords=16384×8 B) |
| Peripheral subsys. | 0x0000_0000_9000_0000 | 0x0000_0000_901F_FFFF | 2 MB | UART, SPI, I2C, GPIO, timers, PLIC |

## 2.3 AXI Bus Parameters

| Parameter | Value | Note |
| --- | --- | --- |
| Data width | 64 bits | matches CVA6 noc_req/noc_resp |
| Address width | 64 bits | v1.0 said 40; corrected to 64 to match CVA6ConfigAxiAddrWidth |
| ID width | 4 bits | as generated by CVA6 |
| User width | 1 bit |  |
| Topology | 1M × 3S | 1 CPU master, 3 slaves |
| Outstanding | 8 per channel | set in axi_xbar parameters |

# 3. CPU Subsystem — CVA6 RV64GC

## 3.1 Core Configuration

| Parameter | Value | Notes |
| --- | --- | --- |
| ISA | RV64GC | G=IMAFD+Zicsr+Zifencei; C=compressed |
| Configuration pkg | cv64a6_imafdc_sv39 | cv64a6_imafdc_sv39_config_pkg |
| Pipeline | 6 stages | Fetch/Decode/Issue/EX/MEM/Commit |
| Issue policy | In-order, single-issue | No OoO in Tier-1 |
| Frequency target | 50 MHz (SKY130), 100 MHz (FPGA) | Synthesis-dependent |
| Physical addr | 39 bits (Sv39 MMU) | 3-level page table |
| Privilege modes | M / S / U | No H-mode in Tier-1 |
| FPU | F + D (IEEE 754 SP+DP) | Zfh deferred |
| AXI interface | noc_req_o / noc_resp_i | CVA6 v5.x port names |
| PMP regions | 8 | M-mode configurable |

## 3.2 L1 Cache Configuration

| Parameter | I-Cache | D-Cache |
| --- | --- | --- |
| Size | 16 KB | 16 KB |
| Associativity | 4-way | 4-way |
| Sets | 64 | 64 |
| Line size | 64 bytes | 64 bytes |
| Replacement | Pseudo-LRU | Pseudo-LRU |
| Write policy | Read-only | Write-back, write-allocate |
| ECC | Parity | Parity |
| Hit latency | 1 cycle | 2 cycles |

# 4. Memory Subsystem

## 4.1 Boot ROM

| Parameter | Value |
| --- | --- |
| Base address | 0x0000_0000_0001_0000 (v1.1 corrected) |
| Size | 64 KB (RomDepth=64 entries × 8 bytes) |
| CVA6 boot addr | 0x0000_0000_0001_0000 |
| Access | Read-only AXI4 slave (SLVERR on writes) |
| Latency | 1 cycle (registered output) |
| Contents | Empty in simulation; ZSBL for ASIC tape-out |

## 4.2 On-Chip SRAM

| Parameter | Value |
| --- | --- |
| Base address | 0x0000_0000_8000_0000 (v1.1 corrected) |
| Default size | 128 KB (SramNumWords=16384, 8 bytes/word) |
| Parameter | SramNumWords — set in t1_soc_top instantiation |
| Word width | 64 bits |
| Byte enables | 8-bit granularity |
| Read latency | 2 cycles (registered) |
| SKY130 macro | sky130_sram_2kbyte_1rw1r_32x512_8 (tiled) |
| FPGA | Xilinx BRAM18 auto-inferred |

# 5. Peripheral Subsystem

All peripherals are sourced from lowRISC OpenTitan (earlgrey_1.0) and connect via t1_periph_ss.sv which bridges AXI4-Lite to TileLink Uncached Lightweight (TL-UL). All registers are 32-bit. The peripheral base address is 0x0000_0000_9000_0000.

## 5.1 Peripheral Address Map

| Peripheral | Offset from 0x9000_0000 | Size | Key Parameters |
| --- | --- | --- | --- |
| UART | 0x0000 | 4 KB | 115200 baud default; 32-deep TX FIFO, 64-deep RX FIFO |
| GPIO | 0x1000 | 4 KB | 32 bidirectional pins; per-pin OE; 32 interrupt sources |
| RV_TIMER | 0x2000 | 4 KB | 2× 64-bit channels; 12-bit prescaler |
| AON_TIMER | 0x3000 | 4 KB | Watchdog: bark→IRQ, bite→SoC reset |
| SPI Host | 0x4000 | 4 KB | 4 chip-selects; SPI modes 0–3; 256-byte TX/RX FIFOs |
| I2C | 0x5000 | 4 KB | Standard/Fast/Fast+; 7/10-bit; 64-byte FIFOs |
| RV_PLIC | 0x6000 | 4 KB | 64 sources; 2 targets (M+S); 8 priority levels |

## 5.2 UART (rv_uart)

| Parameter | Value |
| --- | --- |
| Baud range | 110 – 3,000,000 (NCO = 2^20 × baud / peri_clk) |
| Data format | 8N1 default; configurable stop bits, parity |
| TX FIFO | 32 bytes |
| RX FIFO | 64 bytes |
| Interrupts | 8: TX watermark, RX watermark, TX empty, TX done, RX overflow, RX frame, RX break, RX parity |
| Idle state | uart_tx_o = 1 (marking) when no data |
| Loopback | sys_loopback via CTRL register; line_loopback supported |

## 5.3 SPI Host

| Parameter | Value |
| --- | --- |
| Chip selects | 4 (CS0–CS3); spi_csb_o = 1 idle |
| Max SCK | peri_clk / 2 |
| Modes | SPI 0,1,2,3 (CPOL/CPHA) |
| Width | Standard (1-bit), Dual (2-bit), Quad (4-bit) |
| TX/RX FIFO | 256 bytes each |
| Idle state | spi_csb_o = 1; spi_sck_o = 0 (CPOL=0) |

## 5.4 GPIO

| Parameter | Value |
| --- | --- |
| Pin count | 32 bidirectional (gpio_in_i / gpio_out_o / gpio_oe_o) |
| Direction | Per-pin output enable (gpio_oe_o) |
| Interrupts | 32: rising, falling, both-edge, or level per pin |
| Reset state | gpio_oe_o = 0 (all inputs); gpio_out_o = 0 |

# 6. Clock & Reset Architecture

## 6.1 Clock Domains

| Domain | FPGA source | ASIC source | Frequency | Consumers |
| --- | --- | --- | --- | --- |
| cpu_clk | MMCM output | On-die PLL0 | 50/100 MHz | CVA6, caches, AXI crossbar, SRAM |
| peri_clk | cpu_clk or /2 | PLL0 or /2 | 50 MHz | UART, SPI, I2C, GPIO, PLIC, timers |
| jtag_clk | Ext. TCK pin | Ext. TCK | ≤ 20 MHz | Debug module, DMI JTAG TAP |

## 6.2 Reset Sequence

Step 1: Assert POR (async).
Step 2: PLL0 locks.
Step 3: De-assert peri_rst_n after 4 peri_clk cycles.
Step 4: De-assert cpu_rst_n after 4 cpu_clk cycles.
Step 5: CPU fetches from boot_addr_i = 0x0000_0000_0001_0000.

| Reset | Trigger | Scope | De-assert method |
| --- | --- | --- | --- |
| POR | VDD ramp | Entire SoC | Sync on cpu_clk after PLL lock |
| Soft reset | CPU write CTRL_REG[0] | CPU only, SRAM retained | Sync on cpu_clk |
| WDT bite | 2nd WDT expiry | Entire SoC | Same as POR |
| Debug ndmreset | JTAG ndmreset_o | CPU only (not DM) | Sync on cpu_clk |

# 7. Interrupt Architecture

## 7.1 PLIC Source Assignment

| Source ID | Peripheral | Signal |
| --- | --- | --- |
| 1–8 | UART0 | TX watermark, RX watermark, TX empty, TX done, RX overflow, RX frame, RX break, RX parity |
| 9–10 | SPI0 | Error, SPI event |
| 11–21 | I2C0 | All 11 interrupt sources |
| 22–53 | GPIO[31:0] | Edge / level configurable |
| 54 | rv_timer | Timer 0 expire |
| 55 | aon_timer | Watchdog bark |
| 56–63 | Reserved | — |

# 8. Debug & JTAG

| Parameter | Value |
| --- | --- |
| Spec | RISC-V External Debug Support 1.0 |
| Features | Halt, resume, single-step, 2 HW breakpoints, register/memory access |
| IDCODE | 0x249511C3 |
| TCK max | 20 MHz (FPGA), 10 MHz (SKY130) |
| Abstract commands | Access register (GPR/FPR/CSR), quick access |
| Program buffer | 2 × 32-bit words |
| Software | OpenOCD + riscv64-unknown-elf-gdb |

# 9. Power & Area Estimates (SKY130)

## 9.1 Power

| Block | Dynamic (mW) | Leakage (mW) | Total (mW) |
| --- | --- | --- | --- |
| CVA6 core pipeline | 38 | 22 | 60 |
| CVA6 FPU (active) | 12 | 8 | 20 |
| L1 I-Cache 16 KB | 8 | 10 | 18 |
| L1 D-Cache 16 KB | 9 | 10 | 19 |
| AXI4 crossbar | 4 | 2 | 6 |
| On-chip SRAM 128 KB | 22 | 14 | 36 |
| Boot ROM | 1 | 1 | 2 |
| PLIC + timers | 4 | 2 | 6 |
| Peripheral subsystem | 6 | 3 | 9 |
| Debug module | 0 | 1 | 1 |
| Clock tree + PLL | 15 | 0 | 15 |
| I/O ring (59 pads) | 8 | 4 | 12 |
| Routing margin (15%) | 19 | 11 | 30 |
| TOTAL | 146 | 88 | 234 |

## 9.2 Area

| Block | kGE (cells) | Cell area (mm²) |
| --- | --- | --- |
| CVA6 core pipeline | 280 | 0.403 |
| L1 I-Cache 16 KB | — | 0.112 |
| L1 D-Cache 16 KB | — | 0.112 |
| AXI4 crossbar | 32 | 0.046 |
| Boot ROM (synth.) | 4 | 0.006 |
| On-chip SRAM 128 KB | — | 0.180 |
| PLIC | 18 | 0.026 |
| Peripheral subsystem | 28 | 0.040 |
| Debug module + JTAG | 12 | 0.017 |
| Clock / PLL | 6 | 0.009 |
| I/O ring (59 pads) | — | 0.540 |
| TOTAL | ~380 | ~1.49 mm² |

# 10. RTL File Structure

| File | Module | Description |
| --- | --- | --- |
| rtl/top/t1_soc_top.sv | t1_soc_top | Top integration: CPU, xbar, ROM, SRAM, periph, debug |
| rtl/top/t1_clkrst.sv | t1_clkrst | Clock gating, reset sync, PLL wrapper stub |
| rtl/cpu/cva6_wrapper.sv | cva6_wrapper | CVA6 v5.x parameter wrapper (config_pkg::cva6_cfg_t) |
| rtl/memory/t1_boot_rom.sv | t1_boot_rom | 64 KB synthesized ROM; base 0x0001_0000 |
| rtl/memory/t1_sram_top.sv | t1_sram_top | 128 KB SRAM; base 0x8000_0000 |
| rtl/memory/axi_sram_ctrl.sv | axi_sram_ctrl | AXI4 → SRAM adapter with byte enables |
| rtl/interconnect/t1_xbar.sv | t1_xbar | 1×3 AXI4 crossbar; 64-bit address |
| rtl/peripherals/t1_periph_ss.sv | t1_periph_ss | AXI4-Lite → TL-UL bridge + OpenTitan IPs |
| rtl/peripherals/t1_otp_stub.sv | t1_otp_stub | OTP placeholder (256 B SRAM-backed) |
| rtl/include/prim_*.sv | (stubs) | OpenTitan prim_ stubs for simulation |
| rtl/include/tlul_stubs.sv | (stubs) | TL-UL infrastructure stubs |
| rtl/include/cva6_sim_stubs.sv | (stubs) | instr_tracer + cva6_rvfi_probes stubs |

# 11. Verification Plan

## 11.1 Strategy

| Layer | Scope | Tool | Coverage target |
| --- | --- | --- | --- |
| Unit — SRAM ctrl | AXI-to-SRAM adapter | Verilator + UVM | 100% line, 95% branch |
| Unit — Crossbar | axi_xbar protocol | Verilator + UVM | 100% line, 90% FSM |
| Unit — Peripherals | Each OpenTitan block | Cocotb (Python) | Directed + constrained random |
| Subsystem — CPU | CVA6 ISA compliance | riscv-torture | All RV64GC instructions |
| Formal — Crossbar | AXI4 protocol assertions | SymbiYosys | All SVA assertions proven |
| Integration — Full SoC | All IPs connected | Verilator + UVM | 90% line, 85% branch |

## 11.2 Test Matrix

| Test name | Type | Status | Covers |
| --- | --- | --- | --- |
| t1_tb_top | Smoke | PASS (QuestaSim 2024.1) | 2000-cycle run, no X on outputs |
| t1_reset_tb | Reset stress | PASS | 3× reset cycles, idle-state check |
| t1_gpio_tb | GPIO stimulus | PASS | Walk-1, walk-0, alternating (70 patterns) |
| t1_uart_idle_tb | UART stability | PASS | 5 RX frames + loopback; TX never X |
| t1_spi_idle_tb | SPI idle | PASS | CSB stays high; no X on SPI/UART/GPIO |
| t1_uart_txrx_tb | UART TX/RX E2E | PASS | 98 RX bytes; overflow; loopback; back-to-back; TX framing check |
| t1_sram_test | Constrained random | Planned | All SRAM addresses, bursts, byte strobes |
| t1_burst_test | Constrained random | Planned | AXI4 INCR/FIXED bursts, all sizes |
| t1_periph_test | Directed + random | Planned | UART baud, SPI modes 0–3, I2C, GPIO IRQ |
| t1_irq_test | Directed | Planned | PLIC priority/claim/complete, CLINT, WDT |
| t1_pmp_test | Directed | Planned | PMP violations on locked regions |
| t1_debug_test | Directed | Planned | JTAG halt, resume, single-step, SBA |
| t1_gpio_cpu_tb | CPU-driven (gpio_test.c) | PASS L1 + L2 | UART+GPIO+SPI wiring; GPIO_OUT walk-1/walk-0; GPIO_OE; GPIO_IN=0xA5A5_5A5A readback; L2 adds random patterns |
| t1_uart_cpu_tb | CPU-driven (uart_test.c) | PASS L1 + L2 | UART_CTRL config + readback; TX/RX loopback via TB; parity/framing/overflow error flags; L2 adds 4 extra frames |
| t1_spi_cpu_tb | CPU-driven (spi_test.c) | PASS L1 + L2 | SPI_CTRL config + readback; 4-transfer L1 (0x3C/0xC3/0xFF/0x55); 8-transfer L2 adds 0x0F/0xF0/0x34/0x56; TB SPI slave checks MOSI |
| t1_eclass_gpio_tb (gpio_l1) | E-class GPIO L1 | PASS (QuestaSim 2024.1, 26 Apr 2026) | 16-bit GPIO walk-1/walk-0; boot 0x8000_0000; 83 370 cycles; 0 errors |
| t1_eclass_gpio_tb (gpio_l2) | E-class GPIO L2 | PASS (QuestaSim 2024.1, 26 Apr 2026) | 32-bit GPIO walk-1/walk-0; Boot ROM→SRAM; 75 995 cycles; 0 errors |

# 12. QuestaSim 2024.1 Simulation Setup

## 12.1 Simulation Result (March 2026)

vlog: Errors: 0, Warnings: 165
[TB] Reset released at time 195000
[TB] Simulation completed 2000 cycles — PASS

## 12.2 How to Run

# From project root: OpenSoC_Tier1_Windows_Scripts\

# Compile all RTL:
vlog -work work -sv -f rtl/questa_inc.f -f rtl/questa_src.f

# Run smoke test:
vsim -work work -c t1_tb_top -do "run -all; quit -f"

# Run all tests (batch):
sim\run_all_tests.bat

## 12.3 Known Non-Fatal Issues

| Issue | Impact |
| --- | --- |
| PMP allow_o assertion at time 0 (pmp.sv:89) | Fires before reset; simulation still completes PASS |
| CVA6 reads 0xffffffff80000004 | CVA6 SoC: Boot ROM populated with NOP instructions; CPU fetch succeeds. E-class SoC: fixed — t1_boot_rom_32.sv now uses always_comb case() encoding 3 bootstrap instructions (LUI t0/sp + JALR); vopt dead-code elimination no longer returns 0. |
| vopt-2732 FPGA_EN warnings | Suppressed; FPGA_EN param added to fifo_v3 |
| 165 vlog warnings | All suppressible: FPGA_EN, addr width mismatch remnants |

# Appendix A — Complete Memory Map (Implemented)

| Region | Base | End | Size | Access | Notes |
| --- | --- | --- | --- | --- | --- |
| Boot ROM | 0x0000_0000_0001_0000 | 0x0000_0000_0001_FFFF | 64 KB | RX | CVA6 boot_addr |
| On-chip SRAM | 0x0000_0000_8000_0000 | 0x0000_0000_800F_FFFF | 1 MB | RWX | 128 KB default |
| Peripheral | 0x0000_0000_9000_0000 | 0x0000_0000_901F_FFFF | 2 MB | RW | UART,SPI,I2C,GPIO,timers,PLIC |
| Unmapped | all other | — | — | DECERR | AXI error response |

# Appendix B — I/O Pin List

| Group | Count | Signals | Direction |
| --- | --- | --- | --- |
| Clock | 1 | clk_i | Input |
| Reset | 1 | rst_ni | Input, active-low |
| JTAG | 5 | jtag_tck, tms, trst_n, tdi, tdo + tdo_oe | I/O |
| UART | 2 | uart_rx_i, uart_tx_o | I/O |
| GPIO | 96 | gpio_in_i[31:0], gpio_out_o[31:0], gpio_oe_o[31:0] | I/O |
| SPI | 5 | spi_sck_o, spi_csb_o, spi_sd_i, spi_sd_o, spi_sd_oe_o | I/O |

# Appendix C — IP Repositories

| IP | Repository | Tag/Version |
| --- | --- | --- |
| CVA6 | https://github.com/openhwgroup/cva6 | v5.0.0 |
| PULP AXI4 | https://github.com/pulp-platform/axi | v0.39.0 |
| OpenTitan | https://github.com/lowRISC/opentitan | earlgrey_1.0 |
| common_cells | https://github.com/pulp-platform/common_cells | v0.39.0 |
| OpenLane 2 | https://github.com/The-OpenROAD-Project/OpenLane | v2.2.0 |
| Verilator | https://github.com/verilator/verilator | v5.020 |
| Yosys | https://github.com/YosysHQ/yosys | v0.35 |
| SymbiYosys | https://github.com/YosysHQ/sby | 2024.01 |
| spike | https://github.com/riscv-software-src/riscv-isa-sim | v1.1.0 |
| OpenOCD | https://github.com/riscv-collab/riscv-openocd | v0.12.0 |

# Appendix D — Vendor File Changes for QuestaSim 2024.1

The following files inside rtl/vendor/ were modified to compile and simulate in QuestaSim 2024.1 on Windows. See README_VENDOR_CHANGES.md for full detail.

| File | Change | Error Fixed |
| --- | --- | --- |
| rv_plic/prim_subreg.sv | Added SwAccess enum param, Mubi param, ds output port; case(SwAccess) logic | vopt-2912 |
| rv_plic/prim_subreg_ext.sv | Added ds output port | vopt-2912 |
| rv_plic/rv_plic.sv | Changed prio from unpacked to packed 2D array | vsim-3906 |
| common_cells/fifo_v3.sv | Added parameter bit FPGA_EN = 1'b0 | vopt-2732 |
| common_cells/shift_reg.sv | Replaced with CVA6 version (self-contained always_ff/generate) | Module not found |
| common_cells/addr_decode.sv | Replaced with newer version (contains addr_decode_dync inline) | Module not found |
| rv_uart/uart_core.sv | Added `include "prim_assert.sv" | Undefined macros |
| spi_host/spi_host_fsm.sv | Added `include "prim_assert.sv" | Undefined macros |
| spi_host/spi_host_shift_register.sv | Added `include "prim_assert.sv" | Undefined macros |
| spi_host/spi_host_data_fifos.sv | Added `include "prim_assert.sv" | Undefined macros |
| spi_host/spi_host_window.sv | Added `include "prim_assert.sv" | Undefined macros |
| i2c/i2c_fifo_sync_sram_adapter.sv | Added `include "prim_assert.sv" | Undefined macros |
| i2c/i2c_controller_fsm.sv | Added `include "prim_assert.sv" | Undefined macros |
| i2c/i2c_target_fsm.sv | Added `include "prim_assert.sv" | Undefined macros |
| i2c/i2c_core.sv | Added `include "prim_assert.sv" | Undefined macros |

# Block Diagrams

## Figure 1 — Complete SoC Top-Level Architecture

Figure 1: OpenSoC Tier-1 SoC Top-Level Architecture

## Figure 2 — CVA6 RV64GC Core Architecture

Figure 2: CVA6 RV64GC Core — Pipeline and Functional Units

## Figure 3 — Physical Address Space Map

Figure 3: OpenSoC Tier-1 — Physical Address Space Map

## Figure 4 — AXI4 Crossbar Internal Topology

Figure 4: AXI4 Crossbar Internal Topology

## Figure 5 — Peripheral Subsystem Internal Structure

Figure 5: Peripheral Subsystem Block Diagram

## Figure 6 — Interrupt Architecture

Figure 6: Interrupt Architecture and PLIC Flow

## Figure 7 — Clock and Reset Domain Architecture

Figure 7: Clock and Reset Domain Architecture

# Appendix E — Shakti E-class SoC Variant (Added March 2026)

A 32-bit SoC variant based on the Shakti E-class (RV32IMAC) processor was added to OpenSoC Tier-1 as an alternative to the CVA6 (RV64GC) core. This appendix provides a reference summary. For full details see:

OpenSoC_Tier1_Eclass_Simulation_Report.docx — Verification report

OpenSoC_Tier1_CPU_Comparison_v1.1.docx — Performance/power/area comparison

README_ECLASS.md — Integration guide

rtl/top/t1_soc_top_eclass.sv — SoC top RTL

## E.1 Specification Differences

SPI Master (April 2026 addition): The E-class peripheral subsystem (t1_periph_ss_32.sv) now includes a fully custom SPI master block. It implements SPI Mode 0 (CPOL=0, CPHA=0), MSB-first, 8-bit per transfer, with a configurable SCK half-period (CLK_DIV field, default 4 system clocks). Register map within the peripheral base (0x9000_0000): SPI_CTRL at +0x1C (SPI_EN bit 0, CLK_DIV bits [15:8]), SPI_STAT at +0x20 (SPI_BUSY bit 0, SPI_DONE bit 1, read-only), SPI_TX at +0x24 (write triggers transfer, write-only), SPI_RX at +0x28 (last received byte; reading clears SPI_DONE, read-only). The CPU-driven SPI test (spi_test.c + t1_spi_cpu_tb.sv) verifies PASS at both L1 (4 transfers) and L2 (8 transfers).

| Parameter | CVA6 SoC (Sections 1–14) | E-class SoC (Appendix E) |
| --- | --- | --- |
| CPU ISA | RV64GC (64-bit) | RV32IMAC (32-bit) |
| Pipeline | 6-stage in-order | 3-stage in-order |
| AXI data width | 64-bit | 32-bit |
| AXI masters | 1 (NOC) | 2 (IMEM + DMEM) |
| Boot ROM | 64-bit wide, 64 words (512B) | 32-bit wide, 256 words (1 KB) |
| SRAM | 64-bit wide, 128 KB | 32-bit wide, 128 KB |
| Peripheral SS | Full OpenTitan IP | Custom behavioral stub: UART (TX+RX FSM, 115200 baud), GPIO (32-bit OUT/OE/IN), SPI master (Mode 0, 8-bit, CLK_DIV configurable, SPI_CTRL/STAT/TX/RX registers) |
| FPU | fpnew IEEE 754 | None |
| MMU | Sv39 | None |
| OS support | Linux (with firmware) | RTOS only |
| Work library | work/ | work_eclass/ |
| Regression | 6/6 PASS (CVA6 regression suite) | 7/7 PASS (eclass regression) + 3 CPU-driven (GPIO/UART/SPI L1+L2 PASS) |

## E.2 Address Map (unchanged)

| Region | Base | End | Size |
| --- | --- | --- | --- |
| Boot ROM | 0x0001_0000 | 0x0001_03FF | 1 KB |
| SRAM | 0x8000_0000 | 0x800F_FFFF | 1 MB |
| Peripheral SS | 0x9000_0000 | 0x901F_FFFF | 2 MB |

[NOTE] Address map is identical to CVA6 SoC — software and linker scripts are compatible.

# I2C Master Peripheral

The t1_periph_ss_32 peripheral subsystem includes a CPU-controlled I2C master implemented as a 10-state behavioural FSM in SystemVerilog.  The I2C SCL and SDA signals are exposed as open-drain SoC I/O pins on t1_soc_top_eclass, allowing connection to standard I2C slaves (EEPROMs, sensors, DACs) via external pull-up resistors.

## Register Map

Peripheral base address: 0x9000_0000

| Offset | Register | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x2C | I2C_CTRL | R/W | 0x0000 | [0]=i2c_en, [15:8]=clk_div (SCL half-period in sys-clks) |
| 0x30 | I2C_ADDR | R/W | 0x0000 | [7:0] = (slave_addr<<1)¦rw_bit |
| 0x34 | I2C_TX | W | — | data byte; write triggers one-byte I2C transfer |
| 0x38 | I2C_RX | R | 0x0000 | received data byte; reading clears done flag |
| 0x3C | I2C_STAT | R | 0x0000 | [0]=busy, [1]=done, [2]=nak |

## Transfer Protocol

WRITE transfer: START + (addr<<1|0) + ACK + data_byte + ACK + STOP
READ  transfer: START + (addr<<1|1) + ACK + data_byte(slave) + NACK + STOP

clk_div controls the SCL half-period. At clk_div=4 the SCL frequency is sys_clk / 8, giving ~12.5 MHz SCL at a 100 MHz system clock.

## SoC I/O Ports

| Port | Direction | Description |
| --- | --- | --- |
| i2c_scl_o | output logic | SCL open-drain driver: 0=drive-low, 1=released |
| i2c_scl_i | input logic | SCL wire feedback / clock-stretch input |
| i2c_sda_o | output logic | SDA open-drain driver: 0=drive-low, 1=released |
| i2c_sda_i | input logic | SDA wire feedback / slave read-back |

## Verification Results

| Test | Level | Cycles | Result |
| --- | --- | --- | --- |
| t1_i2c_cpu_tb | L1 (Quick) | 561 | PASS — CTRL readback + 2 write frames (0xA1, 0xA2) to slave 0x50 |
| t1_i2c_cpu_tb | L2 (Full) | 1545 | PASS — 4 writes (0xA1–0xA4) + 2 reads (0xC5, 0x3A) |
| t1_eclass_gpio_tb | L1 (16-bit GPIO) | 83 370 | PASS — GPIO walk-1/walk-0 on 16 pins; TOHOST=0x1; 26 Apr 2026 |
| t1_eclass_gpio_tb | L2 (32-bit GPIO) | 75 995 | PASS — GPIO walk-1/walk-0 on 32 pins; Boot ROM→SRAM; TOHOST=0x1; 26 Apr 2026 |