OpenSoC Tier-1

RTL Design Document

Shakti E-class RV32IM SoC — Module Architecture, Signal Connectivity,
Testbench Structure, and Design Rationale

IIITDM Chennai  ·  OpenSoC Educational SoC Programme  ·  Rev 1.0  ·  April 2026

# 1.  Purpose and Scope

This document describes the RTL implementation of the OpenSoC Tier-1 Shakti E-class SoC. It covers module hierarchy, port connectivity, design decisions, and the simulation testbench architecture for both Level-1 and Level-2 configurations.

The SoC is implemented in synthesisable SystemVerilog and verified in QuestaSim using the rv32i_iss behavioural ISS as the CPU model, allowing C firmware to drive all peripheral verification without requiring a synthesised core.

# 2.  System Architecture

2.1  Compile-Time Configuration Matrix

| Aspect | Level-1 | Level-2 |
| --- | --- | --- |
| Top module | t1_soc_top_eclass | t1_soc_top_eclass |
| Level select define | (none) | `+define+LEVEL2` |
| CPU model (sim) | rv32i_iss (USE_ISS) | rv32i_iss (USE_ISS) |
| Interconnect | t1_bus_l1 (2M×2S) | t1_xbar_32 (2M×3S) |
| SRAM | 4 KB 0x8000_0000 | 128 KB 0x8000_0000 |
| Boot ROM | Absent | t1_boot_rom_32 0x0001_0000 |
| Boot address | 0x8000_0000 | 0x0001_0000 |
| Peripheral subsystem | t1_periph_ss_l1 | t1_periph_ss_32 |
| UART core | OpenTitan uart.sv (NCO baud) | Custom UART (baud_div) |
| GPIO width | 16-bit (OpenTitan gpio.sv) | 32-bit (direct register) |
| SPI master | Absent | Present (8-bit, Mode-0) |
| SPI/GPIO[31:16] ports | Tied off inside top | Exposed on SoC boundary |

2.2  Top-Level Signal Flow

The SoC top (t1_soc_top_eclass.sv) instantiates the CPU, interconnect, memory, and peripheral subsystem.  All inter-block connections are internal wires.  Only clock, reset, UART, GPIO, and SPI signals reach the top-level port list.

AXI4 signal flow (Level-2):

rv32i_iss  ──IMEM_AXI──►  t1_xbar_32  ──S0──►  t1_boot_rom_32

──DMEM_AXI──►             ──S1──►  t1_sram_top_32

──S2──►  t1_periph_ss_32

AXI4 signal flow (Level-1):

rv32i_iss  ──IMEM_AXI──►  t1_bus_l1  ──S0──►  t1_sram_top_32

──DMEM_AXI──►             ──S1──►  t1_periph_ss_l1

# 3.  Module Descriptions

3.1  t1_soc_top_eclass  (rtl/top/t1_soc_top_eclass.sv)

Single canonical SoC top module.  Both Level-1 and Level-2 are selected by the LEVEL2 compile-time define; the port list is identical in both cases. SPI and the upper 16 GPIO bits are tied to 0 internally under Level-1.

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| BootAddr | logic[31:0] | conditional | 0x0001_0000 (L2) or 0x8000_0000 (L1) |

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| clk_i | in | 1 | System clock |
| rst_ni | in | 1 | Active-low synchronous reset |
| uart_tx_o | out | 1 | UART transmit (idle-high) |
| uart_rx_i | in | 1 | UART receive |
| gpio_out_o | out | 32 | GPIO output data (upper 16 tied 0 in L1) |
| gpio_oe_o | out | 32 | GPIO output enable (upper 16 tied 0 in L1) |
| gpio_in_i | in | 32 | GPIO input (upper 16 ignored in L1) |
| spi_csb_o | out | 1 | SPI chip-select bar (tied 1 in L1) |
| spi_sck_o | out | 1 | SPI clock (tied 0 in L1) |
| spi_sd_o | out | 1 | SPI MOSI (tied 0 in L1) |
| spi_sd_i | in | 1 | SPI MISO (unconnected in L1) |

3.2  rv32i_iss  (rtl/cpu/rv32i_iss.sv)

Behavioural RV32IM instruction-set simulator used exclusively in simulation (+define+USE_ISS).  Implements two AXI4 master interfaces (IMEM read-only, DMEM read/write) and drives halt_o on ECALL, EBREAK, illegal opcode, or CSR.

Firmware is loaded at elaboration time via the +HEX_FILE plusarg. The ISS executes instructions internally; all memory accesses are issued over the AXI4 bus so peripheral register writes are real bus transactions visible to the RTL peripheral models.

Note: The ISS has no interrupt or JTAG ports.  timer_irq and ext_irq are not connected to the ISS; they drive the shakti_eclass_wrapper only.

3.3  t1_xbar_32  (rtl/interconnect/t1_xbar_32.sv)  —  Level-2

2-master × 3-slave 32-bit AXI4 crossbar.  Slaves are selected by address decode.  Concurrent IMEM and DMEM transactions to different slaves proceed in parallel. A priority scheme resolves conflicts when both masters target the same slave.

| Slave | Address Range | Module |
| --- | --- | --- |
| S0 — Boot ROM | 0x0001_0000 – 0x0001_FFFF | t1_boot_rom_32 |
| S1 — SRAM | 0x8000_0000 – 0x8001_FFFF | t1_sram_top_32 |
| S2 — Peripheral | 0x9000_0000 – 0x901F_FFFF | t1_periph_ss_32 |

3.4  t1_bus_l1  (rtl/interconnect/t1_bus_l1.sv)  —  Level-1

2-master × 2-slave shared 32-bit AXI4 bus.  DMEM takes priority over IMEM on address conflict.  Simpler decode and lower area than the full crossbar.

| Slave | Address Range | Module |
| --- | --- | --- |
| S0 — SRAM | 0x8000_0000 – 0x8000_0FFF | t1_sram_top_32 (4 KB) |
| S1 — Peripheral | 0x9000_0000 – 0x9000_1FFF | t1_periph_ss_l1 |

3.5  t1_sram_top_32  (rtl/memory/t1_sram_top_32.sv)

Single-port 32-bit wide synchronous SRAM with full AXI4 slave interface.  The NumWords parameter selects depth: 1024 words (4 KB, Level-1) or 32768 words (128 KB, Level-2).  Byte-enable write strobes are supported (WSTRB[3:0]).  TOHOST signalling uses word address 1020 (0x8000_0FF0) by convention.

3.6  t1_boot_rom_32  (rtl/memory/t1_boot_rom_32.sv)  —  Level-2

Read-only 256-word × 32-bit boot ROM mapped at 0x0001_0000.  Contains a minimal three-instruction bootstrap sequence that loads the SRAM base address into sp, sets t0, and executes jalr to transfer control to the SRAM firmware entry point.

lui  t0,  0x80000  # t0 = 0x80000000

lui  sp,  0x80002  # sp = 0x80002000 (stack top in 128 KB SRAM)

jalr x0, t0, 0     # jump to SRAM firmware

3.7  t1_periph_ss_32  (rtl/periph/t1_periph_ss_32.sv)  —  Level-2

Self-contained peripheral subsystem with an internal AXI4-to-register-file bridge. Implements UART (custom baud_div), 32-bit GPIO, and an SPI master (Mode 0, MSB-first). All registers are at offsets 0x00–0x28 within the 0x9000_0000 base window. Unimplemented addresses return 0 on read and are silently ignored on write.

3.8  t1_periph_ss_l1  (rtl/periph/t1_periph_ss_l1.sv)  —  Level-1

Peripheral subsystem for Level-1.  Contains an AXI4-to-TLUL bridge plus OpenTitan vendor IPs: uart.sv (NCO baud generation, 8N1, TX/RX FIFOs) and gpio.sv (16-bit, interrupt-capable).  TLUL packages must be compiled before this module.

Note: The AXI4-to-TLUL bridge state machine uses signals st_q, txn_addr_q, and txn_wr_q which are referenced by the functional coverage module via hierarchical paths.

# 4.  Source File Organisation

| Path | Module / Purpose |
| --- | --- |
| rtl/top/t1_soc_top_eclass.sv | SoC top — single entry point for L1 and L2 |
| rtl/cpu/rv32i_iss.sv | Behavioural RV32IM ISS (simulation CPU) |
| rtl/cpu/shakti_eclass_wrapper.sv | BSV stub wrapper (FPGA/ASIC placeholder) |
| rtl/interconnect/t1_xbar_32.sv | 2M×3S AXI4 crossbar (Level-2) |
| rtl/interconnect/t1_bus_l1.sv | 2M×2S AXI4 shared bus (Level-1) |
| rtl/memory/t1_sram_top_32.sv | Parameterised AXI4 SRAM (shared) |
| rtl/memory/t1_boot_rom_32.sv | Bootstrap ROM (Level-2) |
| rtl/periph/t1_periph_ss_32.sv | Self-contained peripheral SS (Level-2) |
| rtl/periph/t1_periph_ss_l1.sv | OpenTitan-based peripheral SS (Level-1) |
| rtl/vendor/rv_uart/uart.sv | OpenTitan UART IP (L1) |
| rtl/vendor/gpio/gpio.sv | OpenTitan GPIO IP (L1) |
| rtl/include/ | AXI4 type packages and global defines |
| rtl/questa_src_eclass.f | QuestaSim compile filelist |
| verif/tb/t1_eclass_gpio_tb.sv | GPIO testbench (L1 and L2) |
| verif/tb/t1_eclass_uart_tb.sv | UART testbench (L1 and L2) |
| verif/include/tb_level.svh | Testbench macros (timeouts, TOHOST address) |
| sw/tests/gpio_test.c | L1 GPIO firmware |
| sw/tests/gpio_test_l2.c | L2 GPIO firmware |
| sw/tests/uart_test.c | L1 UART firmware |
| sw/tests/uart_test_l2.c | L2 UART firmware |
| scripts/eclass_sim.py | Cross-platform simulation runner |

# 5.  Testbench Architecture

5.1  GPIO Testbench  (verif/tb/t1_eclass_gpio_tb.sv)

Instantiates t1_soc_top_eclass with USE_ISS and LEVEL2 defined as appropriate. Drives gpio_in_i with a known pattern; monitors gpio_out_o and gpio_oe_o. Polls the TOHOST location (0x8000_0FF0) every cycle and terminates on PASS (value 0x1) or FAIL (non-zero, non-one value), or on timeout.

| Macro / Parameter | Level-1 | Level-2 |
| --- | --- | --- |
| Timeout (cycles) | 200 000 | 500 000 |
| HoldCycs | 20 | 30 |
| Boot address checked | 0x8000_0000 | 0x0001_0000 |
| GPIO pattern tested | Walk-1 / walk-0 on 16 pins | Walk-1 / walk-0 on 32 pins |

5.2  UART Testbench  (verif/tb/t1_eclass_uart_tb.sv)

Instantiates the SoC with a loopback connection (uart_tx_o tied to uart_rx_i). Firmware writes a known byte sequence and reads it back; TOHOST is written with PASS on success.  The testbench additionally checks that the TX line returns to idle (high) between frames.

5.3  TOHOST Convention

All testbenches share the TOHOST signalling convention defined in tb_level.svh. The firmware writes a 32-bit value to SRAM word 1020 (0x8000_0FF0). The testbench reads the same SRAM word directly (hierarchical reference) every clock cycle and takes the following action:

| TOHOST value | Testbench action |
| --- | --- |
| 0x0000_0000 | Continue simulation (test not yet complete) |
| 0x0000_0001 | Assert PASS, print cycle count, call $finish |
| Any other non-zero | Assert FAIL with error code, call $finish |
| Timeout reached | Assert TIMEOUT, call $finish with failure |

# 6.  Key Design Decisions

| Decision | Rationale |
| --- | --- |
| Single top file for L1 and L2 | Simplifies maintenance — one module, two defines. Avoids file duplication and keeps the port list stable for FPGA constraint reuse. |
| ISS as simulation CPU | The Shakti E-class BSV core requires a full BSV toolchain to compile. The rv32i_iss provides the same AXI4 bus transactions and allows C firmware to drive peripheral verification without that dependency. |
| Self-contained L2 peripherals vs OpenTitan L1 | L2 (t1_periph_ss_32) prioritises simulation simplicity and no vendor dependency. L1 uses OpenTitan IPs to expose students to a real-world TLUL bus and industry-standard register maps. |
| TOHOST over UART for pass/fail signalling | Direct SRAM polling is cycle-accurate and requires no baud-rate calibration. UART pass/fail is fragile at simulation speed; TOHOST avoids that complexity. |
| NumWords parameter on SRAM | A single parameterised SRAM module serves both levels. The top-level instantiation sets NumWords=1024 (L1) or 32768 (L2). |
| Port list never uses `ifdef | Static port lists simplify Vivado/Quartus wrappers. Tie-offs for L1-absent signals are inside the module body. |

# 7.  Revision History

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | April 2026 | OpenSoC / IIITDM | Initial release. Covers module hierarchy, connectivity, testbench structure, and design rationale for both Level-1 and Level-2 configurations. |