OpenSoC Tier-1

Simulation Verification Report

Shakti E-class RV32IM SoC — QuestaSim Regression Results
GPIO and UART Firmware Tests  ·  Level-1 and Level-2

IIITDM Chennai  ·  OpenSoC Educational SoC Programme  ·  Rev 1.0  ·  April 2026

# 1.  Simulation Environment

| Item | Details |
| --- | --- |
| Simulator | Questa Sim-64 Version 2024.1 (Feb 1 2024) |
| Compile mode | vlog -sv -mfcu (multi-compilation-unit) |
| Optimisation | vopt +acc (separate step, work lib = eclass_iss) |
| Simulation mode | -batch (no GUI) -t 1ps |
| CPU model | rv32i_iss (+define+USE_ISS) |
| Firmware toolchain | riscv-none-elf-gcc / riscv64-unknown-elf-gcc |
| Host OS | Windows 11 via Git Bash / Python 3 |
| Runner | scripts/eclass_sim.py |
| Date of run | 26 April 2026 |

# 2.  Test Plan

| Test ID | Level | Testbench | Firmware | Timeout |
| --- | --- | --- | --- | --- |
| gpio_l1 | Level-1 | t1_eclass_gpio_tb.sv | gpio_test.c → gpio_test_l1.hex | 200 000 cycles |
| gpio_l2 | Level-2 | t1_eclass_gpio_tb.sv | gpio_test_l2.c → gpio_test_l2.hex | 500 000 cycles |
| uart_l1 | Level-1 | t1_eclass_uart_tb.sv | uart_test.c → uart_test_l1.hex | 200 000 cycles |
| uart_l2 | Level-2 | t1_eclass_uart_tb.sv | uart_test_l2.c → uart_test_l2.hex | 500 000 cycles |

# 3.  Results Summary

| Test ID | Level | Result detail | Status |
| --- | --- | --- | --- |
| gpio_l1 | L1 | TOHOST=0x00000001 at cycle 83370 (833795 ns) | PASS |
| gpio_l2 | L2 | TOHOST=0x00000001 at cycle 75995 (760045 ns) | PASS |
| uart_l1 | L1 | Not yet run | PENDING |
| uart_l2 | L2 | Not yet run | PENDING |

# 4.  Detailed Test Results

4.1  gpio_l1  —  PASS

Test: Level-1 GPIO walk-1 / walk-0 pattern over 16 GPIO pins using OpenTitan gpio.sv.

| Item | Value |
| --- | --- |
| Compile library | work_eclass_iss (L1, no LEVEL2) |
| Modules loaded | t1_eclass_gpio_tb, t1_soc_top_eclass, rv32i_iss, t1_bus_l1, t1_sram_top_32, prim_mubi_pkg, prim_secded_pkg, top_pkg, tlul_pkg, t1_periph_ss_l1, uart, gpio |
| HEX file loaded | sw/tests/gpio_test_l1.hex |
| Boot address | 0x8000_0000 (direct SRAM boot, no ROM) |
| TOHOST written at cycle | 83 370 |
| Simulation time | 833 795 ns |
| Wall-clock elapsed | ~2 seconds |
| QuestaSim errors | 0 |
| QuestaSim warnings | 0 |
| TOHOST value | 0x0000_0001 (PASS) |

Verified functionality:

GPIO_OE register write and read-back (all 16 bits set)

GPIO_OUT walk-1 pattern: each bit driven high in sequence, all others low

GPIO_OUT walk-0 pattern: each bit driven low in sequence, all others high

gpio_out_o pin values match GPIO_OUT register (checked in firmware via gpio_in_i loopback)

TOHOST write to 0x8000_0FF0 detected correctly by testbench

4.2  gpio_l2  —  PASS

Test: Level-2 GPIO walk-1 / walk-0 pattern over 32 GPIO pins using t1_periph_ss_32.

| Item | Value |
| --- | --- |
| Compile library | work_eclass_iss (L2, +define+LEVEL2) |
| Modules loaded | t1_eclass_gpio_tb, t1_soc_top_eclass, rv32i_iss, t1_xbar_32, t1_boot_rom_32, t1_sram_top_32, t1_periph_ss_32 |
| HEX file loaded | sw/tests/gpio_test_l2.hex |
| Boot address | 0x0001_0000 (Boot ROM → SRAM) |
| TOHOST written at cycle | 75 995 |
| Simulation time | 760 045 ns |
| Wall-clock elapsed | ~2 seconds |
| QuestaSim errors | 0 |
| QuestaSim warnings | 0 |
| TOHOST value | 0x0000_0001 (PASS) |

Verified functionality:

Boot ROM execution: lui/lui/jalr bootstrap sequence transfers control to SRAM

GPIO_OE register write (full 32-bit width)

GPIO_OUT walk-1 and walk-0 patterns on all 32 pins

gpio_out_o[31:0] values match GPIO_OUT register via firmware loopback

t1_xbar_32 correct slave decode for Boot ROM (S0), SRAM (S1), Peripheral (S2)

TOHOST write to 0x8000_0FF0 detected correctly by testbench

4.3  uart_l1  —  PENDING

Status: UART Level-1 test has not yet been run.  Firmware (uart_test.c) and testbench (t1_eclass_uart_tb.sv) are present.  Run: python scripts/eclass_sim.py uart_l1

4.4  uart_l2  —  PENDING

Status: UART Level-2 test has not yet been run.  Firmware (uart_test_l2.c) and testbench are present.  Run: python scripts/eclass_sim.py uart_l2

# 5.  Functional Coverage

Functional coverage collection is available for Level-2 via the cov_l2 target (python scripts/eclass_sim.py cov_l2 --coverage).  Coverage results are written to build/eclass_cov_l2/coverage_html/index.html.

Note: Coverage run not included in this report.  The coverage target requires the L2 GPIO test to pass first (confirmed above) and adds approximately 10–15 seconds of additional simulation time.

# 6.  Known Issues and Limitations

| ID | Severity | Description | Status |
| --- | --- | --- | --- |
| SIM-001 | Info | Suppressed QuestaSim warnings 2583 (always block sensitivity), 2718 (net driver), 8386 (MBIST). All are benign for behavioural simulation. | Accepted — no functional impact |
| SIM-002 | Info | uart_l1 and uart_l2 tests not yet executed in this run. Firmware and testbenches are present and ready. | Pending — schedule next run |
| SIM-003 | Info | ISS does not model interrupt delivery. timer_irq and ext_irq are driven by the peripheral subsystem but are not connected to rv32i_iss. | By design — ISS limitation; use shakti_eclass_wrapper for IRQ testing |

# 7.  Conclusion

Both GPIO tests (Level-1 and Level-2) pass with zero errors and zero warnings in QuestaSim 2024.1.  The SoC correctly boots from the expected address, executes the GPIO walk-1/walk-0 firmware, and signals PASS via the TOHOST convention. The Level-2 Boot ROM → SRAM transfer is verified end-to-end.

UART tests are pending and should be run to complete the regression. Coverage collection is available but not yet executed.

# 8.  Revision History

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | April 2026 | OpenSoC / IIITDM | Initial report. gpio_l1 and gpio_l2 pass. uart_l1 and uart_l2 pending. |