OpenSoC Tier-1

Functional Specification

Shakti E-class RV32IM SoC — Level-1 and Level-2 Configurations
Interface definitions, functional requirements, memory map, register specification

IIITDM Chennai  ·  OpenSoC Educational SoC Programme  ·  Rev 1.0  ·  April 2026

# 1.  Scope and Purpose

This document specifies the functional behaviour of the OpenSoC Tier-1 Shakti E-class SoC for the IIITDM educational VLSI programme.  It covers both compile-time configurations (Level-1 and Level-2) selected by +define+LEVEL2 at simulation compile time.

The SoC integrates a Shakti E-class RV32IM processor (represented in simulation by the rv32i_iss instruction-set simulator), an AXI4 interconnect, SRAM, Boot ROM (Level-2), and a peripheral subsystem providing UART, GPIO, and SPI.

# 2.  System Overview

| Parameter | Level-1 | Level-2 |
| --- | --- | --- |
| CPU | rv32i_iss (RV32IM ISS) | rv32i_iss (RV32IM ISS) |
| Interconnect | t1_bus_l1 2M×2S 32-bit AXI4 | t1_xbar_32 2M×3S 32-bit AXI4 |
| SRAM | 4 KB at 0x8000_0000 | 128 KB at 0x8000_0000 |
| Boot ROM | None — boots from SRAM | 256×32-bit at 0x0001_0000 |
| Boot address | 0x8000_0000 | 0x0001_0000 |
| Peripheral subsystem | t1_periph_ss_l1 (OpenTitan) | t1_periph_ss_32 (self-contained) |
| UART | OpenTitan uart.sv NCO baud | Custom baud_div register |
| GPIO | 16-bit OpenTitan gpio.sv | 32-bit direct register |
| SPI master | Absent | 8-bit Mode-0 MSB-first |
| I2C | Absent | Absent |
| Level select | Default (no define) | Compile with +define+LEVEL2 |

# 3.  Top-Level Interface

Module: t1_soc_top_eclass  (rtl/top/t1_soc_top_eclass.sv)

3.1  Clock and Reset

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| clk_i | in | 1 | System clock. All registers sample on rising edge. |
| rst_ni | in | 1 | Active-low synchronous reset. Must be held low ≥2 cycles. |

3.2  UART Ports

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| uart_tx_o | out | 1 | UART transmit data. Idle-high (mark state). |
| uart_rx_i | in | 1 | UART receive data. Pull to 1 if unused. |

3.3  GPIO Ports

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| gpio_out_o | out | 32 (L2) / 16 (L1) | GPIO output data register value. |
| gpio_oe_o | out | 32 (L2) / 16 (L1) | GPIO output enable (1=drive, 0=input). |
| gpio_in_i | in | 32 (L2) / 16 (L1) | GPIO input — synchronised internally with 2-FF chain. |

3.4  SPI Ports  (Level-2 only — absent in Level-1 builds)

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| spi_csb_o | out | 1 | SPI chip-select bar. Active-low. High between transfers. |
| spi_sck_o | out | 1 | SPI clock. CPOL=0 CPHA=0 (Mode 0). Idle-low. |
| spi_sd_o | out | 1 | SPI MOSI — master data out, MSB first. |
| spi_sd_i | in | 1 | SPI MISO — slave data in. Sampled on rising SCK. |

Note: SPI and I2C ports are conditionally compiled via `ifdef LEVEL2 in the RTL.  Level-1 builds have no SPI/I2C top-level ports.

# 4.  Memory Map

4.1  Level-2 Address Map

| Region | Base | End | Size | Module |
| --- | --- | --- | --- | --- |
| Boot ROM | 0x0001_0000 | 0x0001_FFFF | 64 KB (256×32) | t1_boot_rom_32 |
| SRAM | 0x8000_0000 | 0x8001_FFFF | 128 KB | t1_sram_top_32 |
| Peripheral SS | 0x9000_0000 | 0x901F_FFFF | 2 MB | t1_periph_ss_32 |

4.2  Level-1 Address Map

| Region | Base | End | Size | Module |
| --- | --- | --- | --- | --- |
| SRAM | 0x8000_0000 | 0x8000_0FFF | 4 KB | t1_sram_top_32 |
| Peripheral SS | 0x9000_0000 | 0x9000_1FFF | 8 KB | t1_periph_ss_l1 |

4.3  TOHOST Signalling Convention

The firmware signals test completion by writing a 32-bit word to SRAM address 0x8000_0FF0 (word index 1020).  The testbench polls this address.

| Value | Meaning |
| --- | --- |
| 0x0000_0000 | Test not yet complete (initial state) |
| 0x0000_0001 | PASS — all firmware checks passed |
| 0xXXXX_YYYY | FAIL — upper 16 bits = error group, lower 16 bits = sub-error code |

# 5.  Peripheral Register Specification

5.1  Level-2 Peripheral Register Map  (t1_periph_ss_32, base 0x9000_0000)

| Offset | Register | Access | Reset | Bit-fields |
| --- | --- | --- | --- | --- |
| 0x00 | UART_CTRL | R/W | 0x0000_0000 | [15:0]=baud_div [16]=uart_en [17]=parity_en |
| 0x04 | UART_STAT | R | — | [0]=tx_empty [1]=tx_full [2]=rx_valid [3]=rx_err |
| 0x08 | UART_TX | W | — | [7:0]=tx_data (write triggers transmission) |
| 0x0C | UART_RX | R | — | [7:0]=rx_data (read clears rx_valid) |
| 0x10 | GPIO_OUT | R/W | 0x0000_0000 | [31:0]=output data register |
| 0x14 | GPIO_OE | R/W | 0x0000_0000 | [31:0]=output enable (1=drive output) |
| 0x18 | GPIO_IN | R | — | [31:0]=synchronised input (2-FF) |
| 0x1C | SPI_CTRL | R/W | 0x0000_0000 | [0]=spi_en [15:8]=clk_div |
| 0x20 | SPI_STAT | R | — | [0]=spi_done [1]=spi_busy |
| 0x24 | SPI_TX | W | — | [7:0]=byte to transmit (write starts transfer) |
| 0x28 | SPI_RX | R | — | [7:0]=received byte (read clears spi_done) |

5.2  Level-1 Peripheral Register Map  (t1_periph_ss_l1, OpenTitan bases)

| Block | Base | Notes |
| --- | --- | --- |
| UART | 0x9000_0000 | OpenTitan uart.sv — NCO-based baud generation. Key regs: CTRL(0x10), STATUS(0x14), WDATA(0x18), RDATA(0x1C). |
| GPIO | 0x9000_1000 | OpenTitan gpio.sv — 16-bit. Key regs: DATA_OUT(0x00), DIRECT_OE(0x10), DATA_IN(0x08). Upper 16 bits tied to 0. |

# 6.  Functional Requirements

6.1  CPU

| ID | Requirement |
| --- | --- |
| CPU-01 | The CPU shall implement the RV32I base integer ISA. |
| CPU-02 | The CPU shall implement the RV32M integer multiplication extension (MUL/DIV/REM). |
| CPU-03 | The CPU shall provide two AXI4 master ports: IMEM (read-only) and DMEM (read/write). |
| CPU-04 | The CPU shall boot from the address specified by the BootAddr parameter. |
| CPU-05 | The CPU shall halt on ECALL, EBREAK, illegal opcode, or any CSR instruction. |
| CPU-06 | In simulation (USE_ISS), the CPU shall load firmware from the HEX_FILE plusarg. |

6.2  Interconnect

| ID | Requirement |
| --- | --- |
| IC-01 | All AXI4 data and address buses shall be 32-bit wide. |
| IC-02 | The interconnect shall decode slave addresses combinatorially with no pipeline stage. |
| IC-03 | The Level-2 crossbar shall support concurrent IMEM and DMEM transactions to different slaves. |
| IC-04 | The Level-1 bus shall give priority to DMEM over IMEM on conflicts. |
| IC-05 | All AXI4 transactions shall complete within the simulation timeout. |

6.3  UART

| ID | Requirement |
| --- | --- |
| UART-01 | The UART shall support 8N1 framing (8 data bits, no parity by default, 1 stop bit). |
| UART-02 | The UART TX line shall be idle-high between frames. |
| UART-03 | The Level-2 UART baud rate shall be clk_freq / (baud_div × 16). |
| UART-04 | The firmware shall be able to read back the UART_CTRL register after write. |
| UART-05 | The Level-2 UART shall support optional even and odd parity (parity_en=1). |

6.4  GPIO

| ID | Requirement |
| --- | --- |
| GPIO-01 | GPIO output data shall be driven on gpio_out_o when gpio_oe_o is asserted. |
| GPIO-02 | GPIO input shall pass through a 2-FF synchroniser before being readable by firmware. |
| GPIO-03 | Level-2 GPIO shall support 32-bit width; Level-1 shall support 16-bit width. |
| GPIO-04 | Firmware shall be able to write walk-1 and walk-0 patterns and read them back. |

6.5  SPI  (Level-2 only)

| ID | Requirement |
| --- | --- |
| SPI-01 | The SPI master shall operate in Mode 0 (CPOL=0, CPHA=0). |
| SPI-02 | Data shall be transmitted MSB-first, 8 bits per transfer. |
| SPI-03 | spi_csb_o shall be asserted (low) only during an active transfer. |
| SPI-04 | The SPI clock frequency shall be clk_freq / (2 × (clk_div+1)). |
| SPI-05 | SPI_STAT[done] shall be set after each complete 8-bit transfer. |

# 7.  Timing Requirements

| Parameter | Value | Notes |
| --- | --- | --- |
| Target clock frequency | 50–100 MHz | Behavioural simulation only — no timing closure required |
| Reset minimum width | 2 rising edges | rst_ni must be low for ≥2 clock cycles |
| AXI4 handshake | Single cycle | All ready/valid handshakes complete in 1 cycle in ISS mode |
| UART baud period (L2) | clk / (baud_div × 16) | Default baud_div=10 → ~520 kbaud at 83 MHz |
| Simulation timeout (L1) | 100 000 cycles | tb watchdog; TOHOST must be written before timeout |
| Simulation timeout (L2) | 500 000 cycles | tb watchdog; TOHOST must be written before timeout |

# 8.  Revision History

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | April 2026 | OpenSoC / IIITDM | Initial release. Covers Level-1 and Level-2 interface specification, memory map, peripheral register maps, and functional requirements. |